PROJECT REPORT

BEng

Phone Myat Thu

Wong Kim Siong

Final Report

Medical Drone for Critical Supply Transport

BACHELOR OF ENGINEERING DEGREE WITH HONOURS IN ROBOTICS AND ARTIFICIAL INTELLIGENCE

Department of Engineering

School of Physics, Engineering and Computer Science

University of Hertfordshire

Medical Drone for Critical Supply Transport

Report by

Phone Myat Thu

Supervisor

Wong Kim Siong

Date

3 April 2026

# DECLARATION STATEMENT

I confirm that the project did not involve human participants or animal experimentation. Accordingly, the ethical approval reference is N/A. The principal project risks were instead electrical, mechanical and operational, including lithium-battery handling, rotating propellers, soldering, and bench-testing of powered flight hardware.

I certify that the work submitted is my own and that any material derived or quoted from the published or unpublished work of other persons has been duly acknowledged. This report has been aligned to the University of Hertfordshire report template and rewritten around the implemented prototype rather than the earlier concept proposal.

Student Full Name: Phone Myat Thu

Student Registration Number:  Number: (UH) pt24aao   |   (PSB) 6205DDDD

Signed: ……………………… …………………………

Date: ………………………3 April 2026 ……………………………

# ABSTRACT

This report presents the design, implementation and preliminary evaluation of a medical-delivery drone prototype developed for critical-supply transport. The implemented system differs substantially from the earlier concept proposal: instead of a larger hexacopter with Raspberry-Pi-centric low-level actuation and broad camera-based autonomy, the realised prototype is a quadrotor built around a Pixhawk 6C flight controller and a Raspberry Pi 5 (8 GB) companion computer. This architectural revision was made deliberately to improve reliability, simplify integration, reduce project risk within the available timeframe, and ensure that the software could demonstrate meaningful mission behaviour rather than remain an ambitious but incomplete concept. The propulsion subsystem uses four 2807 1300 KV brushless motors, Gemfan 9045 propellers, and Hobbywing XRotor 40A ESCs powered from a 4S 1550 mAh 100C LiPo through a Holybro PM02 power module. A separate 3S 550 mAh 95C companion-power battery feeds a 5V 15A UBEC, which in turn powers the Raspberry Pi 5, the direct-GPIO payload servo and the front-mounted 7-inch LCD. This split-power strategy is a key engineering improvement because it reduces exposure of the companion computer to propulsion-induced voltage sag and therefore lowers the risk of Raspberry Pi undervoltage resets during throttle transients. The software platform is based on the v50 Light control application, in which Pixhawk retains low-level flight stabilisation while the Raspberry Pi performs telemetry display, mission-state handling, local waypoint logic, dead-reckoning support, payload sequencing, recording and evidence archiving. A major conceptual clarification made in this report is that the implemented degraded-navigation method is not strict SLAM in the academic sense, because it does not build and optimise a map; instead, it is more accurately described as telemetry-assisted navigation and dead reckoning derived from Pixhawk sensor outputs over MAVLink. The report documents the implemented hardware architecture, engineering calculations for propulsion and power selection, software flow, project-management review, cost analysis, and reasons for changes from the earlier design. Taken together, the results demonstrate a coherent and significantly more defensible engineering prototype than the original outline suggested, while also identifying the measurements and validation still required for full medical-delivery mission readiness.

# ACKNOWLEDGEMENTS

I would like to thank the project supervisor for guidance, laboratory and workshop staff for practical support, and peers and friends who assisted with observations, bench testing and evidence capture. The availability of the supplied report template and the feedback embedded in the earlier marked outline also helped identify the areas requiring greater technical depth and clearer justification.

# TABLE OF CONTENTS

Contents

DECLARATION STATEMENT	iii

ABSTRACT	iv

ACKNOWLEDGEMENTS	v

TABLE OF CONTENTS	vi

LIST OF FIGURES	vii

GLOSSARY	viii

1.	Introduction and background	1

2.	Evolution from the earlier marked outline and rationale for change	3

3.	Literature context and implemented system architecture	5

4.	Engineering design and propulsion / power calculations	8

5.	Software implementation and control logic	13

6.	Implementation evidence, evaluation and discussion	17

7.	Project management, planning and quality-management review	18

8.	Cost analysis, safety, sustainability and regulatory awareness	22

9.	Conclusions and further development	24

REFERENCES	25

BIBLIOGRAPHY	26

APPENDIX A – Weekly Planning	28

APPENDIX B - Figures	29

APPENDIX C – Main Program	32

# LIST OF FIGURES

Appendix Figure B-4  Hardware evidence collage.

Appendix Figure B-3  Interface and archive evidence collage.

Appendix Figure B-2  v50 Light software flowchart.

Appendix Figure B-1  Implemented hardware block diagram.

Figure 5-2  Screenshots of the v50 Light interface and the archived software directory used as evidence of iterative development.

Figure 5-1  Software flowchart for the implemented v50 Light application.

Figure 3-2  Photographic evidence of the implemented prototype, including overall assembly, payload-release area, avionics stack and separate companion-power battery/UBEC assembly.

Figure 3-1  Hardware block diagram of the implemented prototype

# GLOSSARY

Key abbreviations and specialist terms used throughout the report are summarised below.

| Term | Meaning |
| --- | --- |
| AUW | All-up weight of the aircraft, including structure, batteries, avionics and payload. |
| BEC / UBEC | Battery eliminator circuit / universal battery eliminator circuit used to generate regulated low-voltage power from a battery. |
| DR | Dead reckoning. |
| ELRS | ExpressLRS long-range radio protocol used by the receiver/transmitter system. |
| ESC | Electronic speed controller. |
| GNSS / GPS | Global navigation satellite system / Global Positioning System. |
| MAVLink | Message protocol used between the Pixhawk flight controller and companion computer. |
| PM02 | Holybro analog power module used with Pixhawk 6C for voltage/current sensing and regulated flight-controller supply. |
| PWM | Pulse-width modulation, used both for ESC control and servo actuation. |
| SLAM | Simultaneous localisation and mapping; strictly refers to concurrent state estimation and map construction. |
| TX / RX | Transmitter / receiver. |

# Introduction and background

The clinical motivation behind the project is the need for rapid and dependable transport of critical medical supplies in situations where terrestrial logistics are slow, unsafe or unavailable. Examples include emergency medicine dispatch, time-sensitive blood or antivenom delivery, and support to geographically constrained environments where road access is poor. The challenge is not merely to make a drone fly, but to design a system whose architecture is proportionate to the mission: the aircraft must remain controllable and observable, its companion computer must not brown out during power transients, and the payload mechanism must behave deterministically at the target location.

The earlier project outline correctly identified the broader humanitarian relevance of medical-delivery UAVs, but it was based on a different platform concept: a larger hexacopter, a 6S propulsion battery, PCA9685-driven actuation, broad sensor integration and an ambition for camera-based SLAM and onboard AI navigation [1]. Since then, the implemented system has evolved into a more focused and more achievable quadrotor architecture. This report therefore does not attempt to disguise the change; instead, it treats the change as an engineering decision that improved feasibility, reliability and evidential strength.

The final report also needs to satisfy the formal expectations shown in the supplied report template and the final-report guidance slides: technical description alone is insufficient. A strong submission must also demonstrate project management, quality management, awareness of cost and sustainability, and an honest comparison between initial intent and realised outcome [1][2].

1.1 Project aim and revised objectives

The revised aim of the project is to design, integrate and validate a Raspberry-Pi-assisted quadrotor prototype capable of demonstrable waypoint-style mission logic, telemetry-based dead reckoning, payload release and evidence recording, while preserving flight stability through Pixhawk-based low-level control.

To implement a stable hardware architecture centred on a Pixhawk 6C flight controller and a Raspberry Pi 5 companion computer.

To justify the propulsion subsystem using engineering calculations based on battery capability, observed current draw and ESC margin.

To design a split electrical architecture that protects the companion computer from propulsion-bus voltage sag.

To implement a practical companion-computing software stack (v50 Light) with manual control, GPS mission logic, dead reckoning, payload-release sequencing and return-to-start behaviour.

To document and justify the major changes from the earlier project outline in a technically honest and academically defensible way.

To provide a project-management and quality-management review that shows continuous progress rather than only end-state results.

# Evolution from the earlier marked outline and rationale for change

The marked outline that was submitted earlier is useful because it shows the original project intent, but it also highlights the gaps that needed to be corrected in the final report. First, the earlier outline described a 6S hexacopter using a Raspberry-Pi-centric low-level actuation chain through PCA9685 and a wider set of sensors including IR proximity sensors and barometer modules [1]. Second, the marker comments implicitly identified two important weaknesses: the tasks in the plan required more detail, and the report did not adequately explain how medicines would actually be loaded and released [1]. Those points are important because they reveal a common engineering problem: an architecture may look ambitious on paper while still lacking operational clarity.

The realised design departs from the earlier outline in several major ways. The propulsion platform is now a quadrotor rather than a hexacopter. Pixhawk 6C is now clearly responsible for low-level stabilisation and motor control, while the Raspberry Pi is treated as a companion computer rather than as the sole flight controller. The power system is now explicitly split into a propulsion rail and a companion rail. The payload mechanism has become a defined servo-actuated release sequence rather than a placeholder concept. Finally, the navigation strategy has shifted away from always-on camera-based subject/obstacle detection towards telemetry-assisted waypoint logic and dead reckoning.

2.1 Academic explanation of the software-design change

A particularly important change concerns the earlier intention to use the Pi Camera for subject detection, obstacle detection and visual-SLAM-like behaviour. In the implemented system, this approach was deliberately deprioritised. The reason is operational, not merely computational. The intended medical-delivery mission profile is an outdoor, above-ground transport task rather than low-altitude indoor or close-quarter flight in cluttered corridors. At the intended altitude, continuously running human-subject detection or near-field obstacle routines would add computational cost and software complexity while contributing little mission value, because the aircraft is not intended to hover near moving pedestrians or navigate inside buildings. Removing these functions therefore improved mission relevance, reduced false-positive risks, and released processing headroom for the functions that matter more to the current prototype: telemetry monitoring, navigation state logic, payload sequencing and robust user interaction.

The navigation terminology also required correction. Strictly speaking, the implemented degraded-navigation approach should not be described as SLAM. SLAM requires the simultaneous estimation of vehicle state and the construction or optimisation of a spatial map, often with feature association and loop closure. The current prototype does not construct an environment map. Instead, when GNSS is available, it relies on Pixhawk-reported position and heading. When GNSS is degraded or intentionally excluded from the control loop, the Raspberry Pi reads state information exposed by Pixhawk over MAVLink and performs local mission logic and dead reckoning. The academically correct description is therefore telemetry-assisted navigation with dead reckoning, not full SLAM.

2.2 Why the new architecture is stronger

It creates a clearer separation of responsibilities: Pixhawk stabilises; the Raspberry Pi supervises and manages companion-level logic.

It reduces integration risk because the Pi no longer has to guarantee hard real-time motor-mixing performance.

It improves robustness by isolating the companion-power supply from propulsion current transients.

It better matches the available timeframe and therefore produces a more complete demonstrator.

It directly addresses the marker’s need for clearer operational explanation, especially the payload-release mechanism and the work plan detail.

# Literature context and implemented system architecture

3.1 Relevant literature context

Small-UAV design literature consistently emphasises that flight performance is the outcome of coupled airframe, propulsion, control and power choices rather than a single component decision. Beard and McLain [9] discuss the importance of matching controller architecture to platform dynamics and mission objectives, while wider multirotor practice also shows that companion computers are best used for higher-level perception and mission logic when a proven flight controller is available. Likewise, navigation literature distinguishes clearly between GNSS-aided navigation, inertial dead reckoning and full SLAM. That distinction is particularly important here because the implemented prototype uses the first two but not the third.

Therefore, the design philosophy adopted in this project is intentionally layered. Low-level stabilisation remains with Pixhawk 6C, which is designed for deterministic flight-control duties. The Raspberry Pi 5 performs the computationally flexible but less timing-critical tasks: MAVLink telemetry acquisition, graphical user interface rendering, mission-state handling, recording and payload logic. This layered approach is more consistent with good avionics engineering practice than forcing one Linux-based single-board computer to own every level of the control stack.

3.2 Implemented hardware architecture

Figure 3-1 shows the implemented hardware architecture reconstructed from the supplied hardware chart and photographs. The propulsion chain consists of four 2807 1300 KV brushless motors, each driven by its own Hobbywing XRotor 40A ESC. The flight-control chain centres on a Pixhawk 6C connected to GNSS, telemetry and an ELRS receiver. The companion chain centres on a Raspberry Pi 5 (8 GB) connected by USB MAVLink to the Pixhawk, with a Pi Camera Module 3, front 7-inch LCD and a directly controlled payload servo.

Figure 3-1. Hardware block diagram of the implemented prototype

The power architecture is deliberately split. The propulsion subsystem uses a 4S 1550 mAh 100C LiPo feeding the ESCs and the PM02/Pixhawk path. The companion subsystem uses a 3S 550 mAh 95C LiPo feeding a 5V 15A UBEC, which powers the Raspberry Pi 5, the payload servo and the front display. This is not merely convenient wiring; it is a reliability strategy. By separating propulsion and computing power domains, the risk that aggressive throttle changes or ESC noise will collapse the Raspberry Pi rail is substantially reduced.

A direct answer is also now provided to the earlier marker comment asking how medicine is loaded into the drone. In the current prototype, the payload is intended to be carried in a lightweight external container or release cradle integrated with the servo-actuated mechanism. The current build demonstrates the release architecture and command sequence rather than a final medical-grade container. This should be stated honestly in the report: the payload-holding and release logic are implemented, while the containerisation of real medical products remains a next-stage packaging task.

3.3 Hardware evidence from the realised build

Figure 3-2. Photographic evidence of the implemented prototype, including overall assembly, payload-release area, avionics stack and separate companion-power battery/UBEC assembly.

# Engineering design and propulsion / power calculations

4.1 Propulsion selection rationale

The propulsion subsystem uses four 2807 1300 KV motors, Gemfan 9045 propellers and Hobbywing XRotor 40A ESCs on a 4S battery. This combination is consistent with a compact quadrotor that requires strong thrust response and practical current margin without the mass penalty of a larger 6S system. The 2807 stator size provides stronger torque capability than smaller racing-class motors, which helps the motor cope with 9-inch propeller inertia. The 1300 KV constant is also a reasonable compromise for 4S operation: it is low enough to avoid an excessively unloaded speed, yet high enough to preserve responsive control authority.

The Gemfan 9045 propeller choice can be justified qualitatively by disc-area and pitch considerations. A 9-inch diameter increases the actuator disc area relative to smaller 5-inch or 7-inch propellers, which lowers disc loading for a given thrust demand and can improve hover efficiency. The 4.5-inch pitch provides a moderate helical advance, supporting forward-speed capability without forcing the motor into the extreme current draw associated with very aggressive pitch values. In practical terms, this means the 9045 propeller is a reasonable engineering compromise between lift, response and electrical demand for the current airframe.

4.2 Battery capability and current-margin calculations

The propulsion battery is specified as 4S 1550 mAh 100C. Theoretical maximum continuous current based on the label is therefore:

I_max,4S = 1.55 Ah × 100 = 155 A

This figure should not be treated as guaranteed real-world continuous capability, because LiPo C-ratings are often optimistic in practice. Nevertheless, even if the true usable current were materially lower than the label value, the margin relative to the observed operating envelope remains large. The reported real current draw stayed below approximately 35 A during the current operating regime. Expressed as a fraction of the nominal 155 A label capability, this is only about 22.6% of the battery’s theoretical continuous-discharge rating.

Utilisation ratio = 35 / 155 = 0.226 ≈ 22.6%

At the system level, 35 A total current distributed across four propulsion channels corresponds to an average of only 8.75 A per motor/ESC pair:

I_avg per channel = 35 / 4 = 8.75 A

This does not mean each ESC only ever sees 8.75 A, because current distribution becomes uneven during manoeuvre, transient climb and gust rejection. However, it provides a useful average benchmark. Even if a conservative transient multiplier of approximately 3× is applied to account for asymmetry and manoeuvre loading, the implied momentary channel current is about 26.3 A, which remains below the 40 A ESC rating:

I_transient,conservative ≈ 3 × 8.75 = 26.25 A < 40 A

This calculation supports the practical judgement that the 40 A XRotor ESC is sufficient for the present airframe. It is also consistent with Hobbywing’s positioning of the XRotor line as multirotor ESCs with rapid throttle response and software optimised for multirotor motor compatibility [7]. The significance here is not only current capacity, but control behaviour: fast and stable throttle response helps preserve hover quality and manoeuvre precision.

4.3 Hover observation and thrust-margin interpretation

It is obsderved that the vehicle can hover at roughly 40% throttle. This is an important empirical observation because hover-throttle position is a practical indicator of thrust margin. Throttle percentage is not linearly proportional to thrust, and therefore it should not be converted directly into a precise thrust reserve. However, hovering at approximately 40% command does suggest that the propulsion system is not near saturation during steady flight and that there is useful reserve for climb, disturbance rejection and payload release.

This observation is one reason the design is defensible even without a formal thrust-stand dataset. A more complete engineering report in a future iteration should still include measured static-thrust and current curves, but for the current prototype the observed hover point supports the claim that the motor–propeller combination is appropriately sized rather than marginal.

4.4 Endurance estimate from observed current

If the observed system draw of 35 A is used as a representative mission-level current, the idealised endurance of the 4S 1550 mAh battery can be estimated as:

t_ideal = 1.55 / 35 × 60 = 2.66 minutes

Because lithium batteries should not be discharged to 100% in practical flight, a more realistic 80% usable-capacity estimate gives:

t_usable ≈ 0.8 × 2.66 = 2.13 minutes

This number is consistent with the general behaviour of smaller, high-performance quadrotors carrying companion-computing payloads: strong thrust and responsiveness are obtained at the expense of endurance. For the current project stage, that is acceptable because the main objective is systems integration and mission demonstration rather than long-endurance field deployment.

4.5 Companion-power calculation and voltage-sag justification

The companion-power battery is specified as 3S 550 mAh 95C. Its label-based maximum continuous current is therefore:

I_max,3S = 0.55 Ah × 95 = 52.25 A

Again, the C-rating should be treated as an optimistic upper bound, but the figure still indicates substantial headroom relative to the actual companion load. The Pi rail is regulated by a 5V 15A UBEC, giving a nominal maximum power of 75 W. If that full output were ever demanded, the corresponding ideal input current from a 3S pack at 11.1 V would be around 6.76 A before converter losses:

I_in,ideal = (5 × 15) / 11.1 ≈ 6.76 A

In reality, the Raspberry Pi 5, display and servo will usually draw far below this 75 W upper bound, which means the companion battery has ample current margin for the current prototype. The key benefit of the separate 3S + UBEC rail is therefore not simply more available current, but electrical isolation. Raspberry Pi documentation recommends a stable 5V/5A supply for Raspberry Pi 5 [3][4]. By contrast, Holybro documentation lists the PM02 V3 output as 5.2V/3A maximum [5][6]. That difference is highly relevant: powering a Pi 5, display and servo from the PM02 alone would leave significantly less headroom and would make the companion subsystem more vulnerable to rail sag and undervoltage events. The adopted split-power architecture is therefore a strong engineering decision and should be emphasised in the report.

| Parameter | Value | Implication |
| --- | --- | --- |
| 4S propulsion battery capability | 1.55 Ah × 100C = 155 A (label value) | Large theoretical margin relative to the reported <35 A operating draw. |
| Observed system current | <35 A | Supports the claim that propulsion demand remains comfortably below battery and ESC label limits. |
| Average current per propulsion channel | 35/4 = 8.75 A | Shows that average per-ESC loading is far below 40 A during the present operating envelope. |
| Conservative transient channel estimate | ≈26.25 A using 3× average | Still below the 40 A XRotor rating, supporting adequacy with margin. |
| Ideal endurance at 35 A | ≈2.66 min | Short-duration but plausible for a compact high-performance quadrotor. |
| Approximate usable endurance at 80% depth of discharge | ≈2.13 min | More realistic mission figure for present hardware. |
| 3S companion battery capability | 0.55 Ah × 95C = 52.25 A (label value) | Substantial margin for Pi + display + servo via UBEC. |
| UBEC maximum 5V output | 15 A = 75 W | Provides strong headroom for the companion rail and servo transients. |

4.6 Summary table of propulsion and power calculations

| Parameter | Expression / basis | Result | Engineering interpretation |
| --- | --- | --- | --- |
| Motor configuration | 4 × 2807, 1300 KV | 4 motors | Quadrotor propulsion layout |
| Propeller specification | Gemfan CF 9045 | 9 × 4.5 in | Selected for lift-to-efficiency balance |
| ESC rating | 4 × Hobbywing XRotor 40 A | 160 A total rated current | Strong electrical margin relative to reported real current |
| Hover throttle | Experimental observation | ~40% throttle | Indicates the propulsion system is not operating near saturation in hover |
| Propulsion battery | 4S 1550 mAh 100C | 14.8 V nominal, 1.55 Ah | Main flight battery |
| Propulsion battery theoretical max current | 1.55 × 100 | 155 A | Considerably above reported real current demand |
| Propulsion battery theoretical max power | 14.8 × 155 | 2294 W | Indicates ample discharge capability for the selected quadrotor system |
| Reported real system current draw | Measured / observed statement | < 35 A | Supports the claim that the 40 A ESCs are sufficient in this implementation |
| Average current per ESC at 35 A total | 35 / 4 | 8.75 A per ESC | Far below each ESC’s 40 A rating |
| ESC current headroom versus 35 A total | 160 − 35 | 125 A spare total | Large margin for transient load variation |
| ESC loading fraction versus 35 A total | 35 / 160 | 21.9% of combined ESC rating | Confirms conservative ESC sizing |
| Worst-case endurance at 35 A continuous | (1.55 / 35) × 60 | 2.66 min | Conservative lower-bound estimate; actual hover endurance should be better if average current is lower than 35 A |
| Companion battery | 3S 550 mAh 95C | 11.1 V nominal, 0.55 Ah | Dedicated battery for Pi-side electronics |
| Companion battery theoretical max current | 0.55 × 95 | 52.25 A | Sufficient discharge capability for companion electronics |
| Companion battery theoretical max power | 11.1 × 52.25 | 579.98 W | Large theoretical margin at battery side |
| UBEC output capability | 5 V, 15 A | 75 W | Dedicated regulated rail for Raspberry Pi 5, servo, camera and LCD |
| Raspberry Pi 5 recommended supply | Official documentation basis | 5 V / 5 A | Justifies use of dedicated high-current regulated companion rail |
| Remaining UBEC current after reserving 5 A for Pi 5 | 15 − 5 | 10 A | Provides margin for front display, servo actuation and transient loads |
| Power architecture | Split supply | Main battery for propulsion, separate companion battery + UBEC for Pi-side electronics | Reduces voltage sag coupling and lowers risk of Raspberry Pi undervoltage or reboot during throttle changes |

# Software implementation and control logic

5.1 Software architecture overview

The implemented software baseline is the v50 Light control application. This software is designed around a companion-computing philosophy: it does not replace the Pixhawk flight stack. Instead, it performs state acquisition, user interaction and mission logic while sending only local stick-style override commands when the transmitter override switch is active. The key software modules are:

- MAVLink communication worker for heartbeat detection and telemetry acquisition.

- Graphical user interface (PyQt) for dashboard, GPS and dead-reckoning tabs.

- Manual keyboard-control logic for roll, pitch and yaw stick simulation.

- Mission-state logic for GPS waypointing and dead reckoning.

- Payload servo logic implemented directly from Raspberry Pi GPIO rather than via Pixhawk.

- Optional recording and implementation-evidence archiving.

- Return-to-start sequencing after payload release.

Figure 5-1. Software flowchart for the implemented v50 Light application.

5.2 Detailed explanation of mission logic

The software first connects to Pixhawk over MAVLink, waits for heartbeat confirmation, and requests the telemetry streams required by the interface. These include attitude, global position, heading, groundspeed, altitude and RC switch state. The software then operates in one of three broad modes: idle/manual, GPS mission mode, or dead-reckoning mode.

In idle/manual mode, the Raspberry Pi functions as a human-operated companion interface. Keyboard commands adjust simulated stick targets, while the space bar centres the active stick channels. The payload servo can be moved directly using keyboard commands or the GUI buttons.

In GPS mission mode, the user selects a target from the map tab. The Raspberry Pi reads Pixhawk-reported latitude, longitude and heading, computes the heading error to the target, and generates only local stick-style roll/pitch/yaw corrections. This is an important implementation detail: the Pi does not attempt to reconfigure Pixhawk flight mode from the companion side. It computes the navigation correction locally and expresses it as if a human operator were moving the sticks.

In dead-reckoning mode, the Pi uses heading, groundspeed, elapsed time and state variables reported from Pixhawk to estimate motion when direct waypoint progression is unavailable or intentionally not used. Again, the logic remains local to the companion computer, but the aircraft state foundation is the Pixhawk telemetry stream. This makes the software easier to justify than a loosely defined “AI navigation” claim because the actual information flow is explicit and testable.

5.3 Payload release and return sequence

A major improvement over the earlier outline is that the payload mechanism is no longer left as an abstract idea. In the current implementation, the payload sequence is defined explicitly. When the aircraft reaches the target condition, the GPIO-driven servo opens the release mechanism, remains open for five seconds, then closes. After a further five-second dwell, the software sets the mission start point as the return target and begins the return-to-start leg. This sequence gives the report a much stronger operational narrative because it clearly explains what the aircraft does at destination rather than simply stating that a payload exists.

5.4 Why subject detection and obstacle detection were removed

The earlier software direction included subject detection and obstacle-detection ideas using the Pi Camera. These were intentionally removed from the core mission software in the current design. This was not a reduction in seriousness; it was a refinement of mission relevance. The aircraft is intended to operate as an outdoor delivery vehicle at altitude rather than as a low-flying indoor inspection platform or a consumer-style obstacle-avoidance drone. The probability of encountering near-field obstacles or “flying humans” in the intended operating envelope is low compared with the cost of running continuous vision inference. Removing these functions improves timing determinism, lowers CPU load on the Raspberry Pi, reduces sources of false alarms, and focuses the software on the functions that actually matter to the current prototype.

5.5 Software evidence and archive traceability

Figure 5-2 shows evidence of the implemented interface and the archived version history. The Google Drive screenshot is particularly useful because it demonstrates that the software evolved through multiple saved versions rather than a single one-off script. For project assessment, this matters because it evidences week-on-week progression, iterative debugging and configuration management discipline.

Figure 5-2. Screenshots of the v50 Light interface and the archived software directory used as evidence of iterative development.

# Implementation evidence, evaluation and discussion

6.1 What has been implemented successfully

- A physically integrated quadrotor prototype with Pixhawk 6C, Raspberry Pi 5, companion display and payload-release servo.

- A split electrical architecture using separate propulsion and companion-power batteries.

- A working GUI with dashboard, GPS and dead-reckoning tabs.

- Manual keyboard stick simulation linked to the transmitter override switch.

- GPIO-based payload servo control and mission-stage release sequence.

- Version archiving and evidence collection through screenshots and photographs.

6.2 Discussion of strengths and limitations

The strongest aspect of the prototype is architectural coherence. The current build no longer reads like a list of ambitious subsystems that have yet to be reconciled; it now behaves like a layered UAV system with a clear flight-control chain, a clear companion-computing chain and a clear power-distribution rationale. This is exactly the kind of improvement that turns a mid-range report into a high-quality engineering report.

The main limitation is that some quantitative validation remains incomplete. The propulsion calculations are supported by observed current and hover behaviour, but the report would be even stronger with exact measured all-up weight, thrust-stand curves, current logs and endurance data from repeated tests. Likewise, the return-to-start and degraded-navigation logic are logically implemented, but their field reliability still depends on future supervised flight validation.

6.3 Why the revised design is more appropriate than the original

From an engineering-management perspective, the most important achievement may be the willingness to make major design changes instead of defending an outdated concept. The earlier concept was interesting, but it spread the project across too many simultaneous technical risks: a larger airframe, a Pi-owned low-level control path, broad sensor fusion, obstacle sensing and camera-based autonomy. The revised design reduced scope without abandoning ambition. It preserved the core research direction—medical delivery with Raspberry-Pi-assisted mission logic—while moving the lower-level flight responsibilities back to the subsystem best suited for them: Pixhawk.

# Project management, planning and quality-management review

7.1 Marker feedback and how the final report responds

The earlier marked outline already hinted at the areas that needed improvement in the final report. Most notably, the marker noted that the task descriptions required more detail and asked how the medicine would be loaded into the drone [1]. This final report responds directly to those points by providing: (i) a week-by-week implementation plan; (ii) a defined payload-loading and release narrative; (iii) a much clearer separation between concept and implementation; and (iv) a more quantitative engineering section.

7.2 Detailed week-by-week implementation plan

| Week | Primary work package | Detailed tasks | Expected evidence / deliverables |
| --- | --- | --- | --- |
| 1 | System bring-up and architecture freeze | Confirm final quadrotor architecture; verify Pixhawk–Pi USB link; power up PM02 and companion rail separately; create initial file archive and naming structure. | Heartbeat confirmation, initial wiring validation, archived project folder. |
| 2 | Telemetry ingestion and interface skeleton | Read heartbeat, attitude, GPS and RC channels over MAVLink; create status strip and tab framework in PyQt; confirm override-switch state display. | Working telemetry dashboard prototype. |
| 3 | Manual local-stick control | Implement keyboard roll/pitch/yaw adjustment; centre-stick function; safe override release; transmitter override-switch gating. | Manual control bench demo and screenshots. |
| 4 | GPS mission interface | Integrate map tab, target selection and distance display; implement local heading-error guidance logic. | Waypoint UI page and GPS screenshots. |
| 5 | Dead-reckoning logic | Implement bearing/distance mode and coordinate mode; compute local travel estimate from telemetry; expose DR status and progress. | Dead-reckoning interface and logic screenshots. |
| 6 | Servo and payload mechanism integration | Move payload actuator to Raspberry Pi GPIO control; test servo open/mid/close states; mount release mechanism physically. | GPIO servo control working, payload close-up photographs. |
| 7 | Mission sequencing | Implement arrival condition, 5 s open, close, wait and return-to-start sequence for GPS and DR missions. | Integrated mission-state software evidence. |
| 8 | Recording and archive evidence | Add recording option and organise version-controlled saved scripts and screenshots in cloud/local folders. | Archived software versions, recording screenshots. |
| 9 | UI cleanup and light-theme polish | Improve readability, tab structure, labels and state presentation; reduce clutter and remove less relevant modules. | v50 Light software baseline. |
| 10 | Bench validation and fault fixing | Fix state/abort bugs; verify servo status; verify ch9 override behaviour; test Pi power stability with companion rail. | Stable demonstration build. |
| 11 | Documentation generation | Prepare block diagrams, software flowchart, BOM table, engineering calculations and figure captions. | Draft report figures and tables. |
| 12 | Final report completion | Write reflective discussion, quality-management review, references and appendices; align final report with template and marking expectations. | Submission-ready final report draft. |

Table 7-1 provides the detailed plan that the earlier outline lacked. It is intentionally written in an implementation-oriented form rather than as vague milestone labels.

7.3 Comparison between initial plan and realised plan

The difference between the original plan and the realised plan is not a sign of failure; it is evidence that the project was actively managed. The initial plan assumed simultaneous progress on many high-risk threads. The realised plan narrowed the architecture so that each week could produce visible system progress. This made the project more robust against time overrun and also improved the eventual report quality, because each major claim could be supported by implementation evidence.

7.4 Reflective review: strengths, weaknesses and what would be changed

Strengths of the project management included initiative, persistence, and effective iteration through many software versions. The most visible weakness was initial scope optimism. In hindsight, the project would have benefited from a formal validation matrix from the first month, with planned measurement points for weight, current, endurance and payload mass.

What went well: architecture revision, power-domain separation, iterative software development, evidence archiving, and practical integration of payload logic.

What could be improved: earlier test planning, more disciplined cost logging, earlier quantitative measurements, and earlier definition of the payload container.

What would be done differently if restarted: freeze the airframe earlier, log every component receipt, create a requirements-to-evidence matrix from the start, and schedule dedicated measurement weeks for propulsion and power data.

7.5 Quality management and continuous improvement

Quality management in this project was reflected not by formal certification, but by engineering behaviours: version archiving, modular architecture, explicit separation of power domains, progressive bug fixing and evidence-based reporting. These behaviours align with the quality-management questions provided for the final report because they show how the process was controlled rather than merely how the product looked at the end.

The project also demonstrates that quality and scope control are linked. Once non-essential functions were removed from the primary mission path—especially always-on subject detection and obstacle logic—the software became simpler, more deterministic and easier to validate. That simplification is itself a quality improvement because it reduces the number of failure paths that can interrupt a mission-critical delivery sequence.

# Cost analysis, safety, sustainability and regulatory awareness

8.1 Bill of materials and cost summary

| Item | Qty | Approx. unit price (SGD) | Approx. subtotal (SGD) | Price basis |
| --- | --- | --- | --- | --- |
| Pixhawk 6C + GPS + PM02 package | 1 | 330 | 330 | Singapore Hobby Supplies |
| Telemetry radio set | 1 | 80 | 80 | Singapore Hobby Supplies |
| Radiomaster ELRS receiver | 1 | 30 | 30 | Hobby Square |
| Radiomaster TX15 Max transmitter | 1 | 260 | 260 | Hobby Square |
| Raspberry Pi 5 8 GB | 1 | 130 | 130 | Kuriousity |
| Front 7-inch LCD | 1 | 85 | 85 | Kuriousity |
| Pi Camera Module 3 | 1 | 45 | 45 | Shop from SimLim Tower |
| Hobbywing XRotor 40A ESC | 4 | 20 | 80 | Hobby Square |
| 2807 1300 KV motor | 4 | 20 | 80 | Hobby Square |
| Gemfan 9045 propellers | 2 pairs | 6 | 12 | Hobby Square |
| 4S 1550 mAh 100C LiPo | 1 | 45 | 45 | Hobby Square |
| 3S 550 mAh 95C LiPo | 1 | 15 | 15 | Hobby Square |
| 5V 15A UBEC | 1 | 25 | 25 | Shopee |
| Payload servo | 1 | 10 | 10 | Shopee |
| Frame, wiring, fasteners, connectors, printed parts | 1 lot | 140 | 140 | Hobby Square + Mechanical Lab from School + SimLim Tower |
| Approximate total |  |  | 1,367 |  |

8.2 Safety engineering

The prototype involves electrical, mechanical and operational hazards. The most obvious are lithium-battery handling, rotating propellers and accidental throttle activation during bench testing. Accordingly, the project should document the following controls explicitly: propellers removed during software bring-up; separate continuity and polarity checks before first power application; isolated battery connection during soldering; secure mounting of both propulsion and companion batteries; and supervised indoor bench tests only at low risk levels. These controls should be included because the earlier appendices already indicated the need for formal risk assessment [1].

8.3 Sustainability and practical deployment relevance

Sustainability in this context is not limited to electrical efficiency. It also concerns whether the design is fit for repeated use, maintainable and appropriately modular. The split-power architecture improves serviceability because propulsion and computing subsystems can be isolated during fault finding. Likewise, the companion-computing approach allows the UI and mission logic to evolve without replacing the whole flight-control stack. For a humanitarian delivery concept, this modularity is practically valuable because it reduces replacement cost and speeds up iteration.

8.4 Regulatory awareness

Any outdoor operation in Singapore must comply with Civil Aviation Authority of Singapore (CAAS) requirements, including registration and any permit or training obligations that apply to the aircraft weight and operating area [8]. The present report does not claim regulatory clearance; rather, it shows awareness that technical feasibility and legal feasibility are inseparable for medical-delivery UAV work.

# Conclusions and further development

The final report is significantly stronger than the earlier outline because it now describes the system that was actually built. The implemented prototype is not a finished medical-delivery product, but it is a coherent engineering demonstrator with justified propulsion selection, a sensible split-power architecture, a clarified companion-computing control philosophy, a defined payload-release mechanism and visible evidence of iterative software development. Most importantly, the project no longer depends on inflated claims about SLAM or unnecessary obstacle-detection functions. Instead, it presents a more rigorous and more believable story: Pixhawk provides flight stability, the Raspberry Pi supervises mission behaviour, and the overall architecture is aligned with what  was realistically able to integrate and demonstrate.

The strongest next steps are now clear. The project should measure and report exact all-up weight, payload mass, hover current, return-flight current and usable endurance; add formal outdoor validation under the relevant approvals; refine the medicine container and release fixture; and, if desired, extend the degraded-navigation layer with better state estimation. If these next-stage activities are completed systematically, the platform will move substantially closer to a fully defensible medical-delivery UAV demonstrator.

9.1 Recommended next-stage work

- Record exact all-up weight, payload mass and centre-of-gravity position.

- Capture logged current and voltage data on both the propulsion and companion rails.

- Validate repeatable payload-release timing and latch reliability across multiple trials.

- Perform outdoor waypoint and return-to-start tests under the relevant safety permissions.

- Refine degraded-navigation estimation using better sensor-fusion methods if GPS becomes unavailable.

- Develop a dedicated medical payload box with retention, protection and release safety features.

# REFERENCES

[1] Phone Myat Thu (2025) Project Outline: Medical Drone for Critical Supply Transport. Undergraduate Project Appendices, University of Hertfordshire. The marked outline included feedback requesting more detailed tasks and a clearer explanation of how medicine is loaded/released.

[2] University of Hertfordshire (undated) Project Report Template. School of Physics, Engineering and Computer Science, University of Hertfordshire.

[3] Raspberry Pi Ltd. (2026) Getting started – Raspberry Pi Documentation. Available from the official Raspberry Pi documentation portal. States that Raspberry Pi 5 is recommended for a 5V/5A supply and that lower-power supplies restrict available peripheral current.

[4] Raspberry Pi Ltd. (2026) Raspberry Pi 5 Product Brief. Official product brief specifying 5V/5A DC power via USB-C.

[5] Holybro (2025) Power Module Comparison. Official Holybro documentation comparing PM02 V3, PM06 V2 and related power modules; lists PM02 V3 output as 5.2V/3A max.

[6] Holybro (2025) PM02 V3 documentation. Official Holybro documentation for the analog power module family used with Pixhawk-class controllers.

[7] Hobbywing (2026) XRotor 40A 6S BLDC product information. Official Hobbywing product information describing rapid throttle response and multirotor motor compatibility.

[8] Civil Aviation Authority of Singapore (2025) UA Regulatory Requirements / UA Registration pages. Official CAAS guidance describing Singapore unmanned-aircraft requirements.

[9] Beard, R. and McLain, T. (2012) Small Unmanned Aircraft: Theory and Practice. Princeton University Press.

[10] Groves, P.D. (2013) Principles of GNSS, Inertial, and Multisensor Integrated Navigation Systems. 2nd edn. Artech House.

[11] Cadena, C. et al. (2016) “Past, present, and future of simultaneous localization and mapping: Towards the robust-perception age”, IEEE Transactions on Robotics, 32(6), pp. 1309–1332.

# BIBLIOGRAPHY

ArduPilot (no date) ArduPilot Copter. Copter documentation. Available at: https://ardupilot.org/copter/

ArduPilot (no date) First Time Setup. Copter documentation. Available at: https://ardupilot.org/copter/docs/initial-setup.html

ArduPilot (no date) Complete Parameter List. Copter documentation. Available at: https://ardupilot.org/copter/docs/parameters.html

ArduPilot (no date) Choosing an Autopilot. Copter documentation. Available at: https://ardupilot.org/copter/docs/common-autopilots.html

Holybro (2024) Pixhawk 6C. Holybro documentation. Available at: https://docs.holybro.com/autopilot/pixhawk-6c

Holybro (2025) Overview. Pixhawk 6C documentation. Available at: https://docs.holybro.com/autopilot/pixhawk-6c/overview

Holybro (2024) Pixhawk 6C Ports. Holybro documentation. Available at: https://docs.holybro.com/autopilot/pixhawk-6c/pixhawk-6c-ports

Holybro (2024) Power Module. Holybro documentation. Available at: https://docs.holybro.com/power-module-and-pdb/power-module

Holybro (2024) Digital Power Module (PM) Setup. Holybro documentation. Available at: https://docs.holybro.com/power-module-and-pdb/power-module/digital-power-module-pm-setup

MAVLink (no date) MAVLink Developer Guide. MAVLink Guide. Available at: https://mavlink.io/en/

MAVLink (no date) Protocol Overview. MAVLink Guide. Available at: https://mavlink.io/en/about/overview.html

MAVLink (no date) MAVLINK Common Message Set (common.xml). MAVLink Guide. Available at: https://mavlink.io/en/messages/common.html

Raspberry Pi Ltd (no date) Raspberry Pi Documentation. Available at: https://www.raspberrypi.com/documentation/

Raspberry Pi Ltd (no date) Getting started. Raspberry Pi Documentation. Available at: https://www.raspberrypi.com/documentation/computers/getting-started.html

Raspberry Pi Ltd (2026) Raspberry Pi 5 product brief. Available at: https://pip.raspberrypi.com/documents/RP-008348-DS-raspberry-pi-5-product-brief.pdf

Raspberry Pi Ltd (no date) Camera. Raspberry Pi Documentation. Available at: https://www.raspberrypi.com/documentation/accessories/camera.html

Raspberry Pi Ltd (no date) Camera software. Raspberry Pi Documentation. Available at: https://www.raspberrypi.com/documentation/computers/camera_software.html

Raspberry Pi Ltd (no date) Raspberry Pi OS. Available at: https://www.raspberrypi.com/software/

# APPENDIX A – Weekly Planning

| Week | Summary |
| --- | --- |
| 1 | Freeze architecture; verify Pixhawk–Pi communications and separate power rails. |
| 2 | Implement telemetry worker and dashboard framework. |
| 3 | Implement manual local-stick control and safe override release. |
| 4 | Implement GPS interface and local waypoint logic. |
| 5 | Implement dead-reckoning modes. |
| 6 | Integrate GPIO payload servo and physical release mechanism. |
| 7 | Add arrival sequencing and return-to-start logic. |
| 8 | Organise archive structure and recording/evidence capture. |
| 9 | Refine light-theme UI and remove less relevant modules. |
| 10 | Bench test and fault-fix control logic. |
| 11 | Prepare figures, calculations, tables and narrative. |
| 12 | Complete final report, reflection and appendices. |

This appendix repeats the detailed weekly work plan in a compact form to make it easy to cross-reference against the project-management review.

# APPENDIX B - Figures

Appendix Figure B-1. Implemented hardware block diagram.

Appendix Figure B-2. v50 Light software flowchart.

Appendix Figure B-3. Interface and archive evidence collage.

Appendix Figure B-4. Hardware evidence collage.

# APPENDIX C – Main Program

import os

import sys

import time

import math

import threading

from datetime import datetime

import numpy as np

import cv2

from picamera2 import Picamera2

from pymavlink import mavutil

from PyQt5.QtWidgets import (

QApplication, QMainWindow, QWidget, QTabWidget,

QVBoxLayout, QHBoxLayout, QGridLayout,

QLabel, QPushButton, QLineEdit, QSpinBox,

QProgressBar, QRadioButton, QButtonGroup,

QGroupBox, QFrame, QSizePolicy, QMessageBox,

)

from PyQt5.QtCore import (

Qt, QTimer, QUrl, QRectF,

pyqtSignal, QObject, QThread,

)

from PyQt5.QtGui import (

QPainter, QColor, QPen, QFont, QBrush,

QPalette, QLinearGradient, QImage, QPixmap,

QConicalGradient,

)

from PyQt5.QtWebEngineWidgets import QWebEngineView, QWebEnginePage

# ─────────────────────────────────────────────────────────────

#  CONFIG  — edit these to match your setup

# ─────────────────────────────────────────────────────────────

PORT = "/dev/serial/by-id/usb-Holybro_Pixhawk6C_29004B001051333235363832-if00"

# Override switch channel

OVERRIDE_SWITCH_CH = 9          # TX channel number (1-indexed)

SWITCH_LOW_PWM     = 1300       # below this = switch OFF

SWITCH_HIGH_PWM    = 1700       # above this = switch ON

# PWM constants

NO_OVERRIDE  = 65535

NEUTRAL      = 1500

MIN_PWM      = 1100

MAX_PWM      = 1900

STEP         = 10

SEND_HZ      = 20

UI_HZ        = 10

MAX_PWM_RATE = 400              # max PWM change/sec for smooth stick

# Face safety

AUTO_CLOSE_RATIO  = 0.18

AUTO_SAFE_RATIO   = 0.15

PITCH_BACKOFF_PWM = 1200

PITCH_NEUTRAL_PWM = 1500

# GPS autopilot (stick-nudge)

WAYPOINT_RADIUS_M = 3.0

AP_YAW_KP         = 2.0        # deg error → extra PWM

YAW_DEADBAND_DEG  = 8.0

AP_PITCH_FWD      = 1600       # forward pitch PWM toward target

# Dead reckoning

DR_HZ              = 10

CRUISE_SPEED_MPS   = 2.0

DR_MIN_GS_VALID    = 0.2

YAW_ALIGN_SEC      = 1.0

DR_PROGRESS_RADIUS_M = WAYPOINT_RADIUS_M

# Camera

TARGET_FPS     = 60

DETECT_EVERY   = 2

DETECT_SCALE   = 0.30

MIN_FACE_SMALL = (60, 60)

BOX_TIMEOUT    = 0.6

SMOOTH_BOX     = 0.55

# Camera colour path (v11.3)

# Pi Camera Module 3 NoIR on some Pi5 builds reports "BGR888" but the

# captured bytes are already in the order Qt expects for RGB888 preview.

# That means an extra cv2 BGR→RGB conversion can create the exact symptom:

# blue face / orange shirt.

#

# Default for this file: DO NOT swap channels.

# If your own setup still shows reversed colours, set CAMERA_SWAP_RB = True.

CAMERA_NATIVE_FORMAT = "BGR888"

CAMERA_SWAP_RB      = False

SHOW_COLOR_SWATCHES = True

# Scene identification / recording

ID_EVERY = 4

HOG_SCALE = 0.50

VEGETATION_GREEN_RATIO = 0.18

RECORD_DIR = os.path.expanduser("~/Videos/drone_gcs_recordings")

# Servo / payload release (direct Raspberry Pi GPIO servo)

# Default signal pin: BCM GPIO18  (physical pin 12)

# Recommended power: external regulated 5V supply + common ground with Pi.

SERVO_GPIO_PIN = 18

SERVO_OPEN_PWM = 1900

SERVO_MID_PWM = 1500

SERVO_CLOSE_PWM = 1100

PAYLOAD_OPEN_SEC = 5.0

PAYLOAD_WAIT_SEC = 5.0

# Map defaults

DEFAULT_MAP_LAT  = 3.1390

DEFAULT_MAP_LON  = 101.6869

DEFAULT_MAP_ZOOM = 16

# ─────────────────────────────────────────────────────────────

#  COLOUR PALETTE

# ─────────────────────────────────────────────────────────────

C_BG       = "#f5f7fb"

C_SURFACE  = "#ffffff"

C_CARD     = "#eef3f9"

C_BORDER   = "#cfd8e3"

C_ACCENT   = "#2f7de1"

C_GREEN    = "#2e9b45"

C_YELLOW   = "#b7791f"

C_RED      = "#d64545"

C_CYAN     = "#1f8f83"

C_TEXT     = "#1f2937"

C_DIM      = "#5f6b7a"

C_ORANGE   = "#d97706"

# ─────────────────────────────────────────────────────────────

#  HELPERS

# ─────────────────────────────────────────────────────────────

def clamp(v):

return max(MIN_PWM, min(MAX_PWM, int(v)))

def haversine_m(lat1, lon1, lat2, lon2):

R = 6_371_000.0

p1, p2 = math.radians(lat1), math.radians(lat2)

dp = math.radians(lat2 - lat1)

dl = math.radians(lon2 - lon1)

a = math.sin(dp / 2) ** 2 + math.cos(p1) * math.cos(p2) * math.sin(dl / 2) ** 2

return R * 2 * math.atan2(math.sqrt(a), math.sqrt(1 - a))

def bearing_to(lat1, lon1, lat2, lon2):

la1, la2 = math.radians(lat1), math.radians(lat2)

dlo = math.radians(lon2 - lon1)

x = math.sin(dlo) * math.cos(la2)

y = math.cos(la1) * math.sin(la2) - math.sin(la1) * math.cos(la2) * math.cos(dlo)

return (math.degrees(math.atan2(x, y)) + 360) % 360

def gps_offset(lat, lon, bearing_deg, dist_m):

R = 6_371_000.0

b = math.radians(bearing_deg)

la = math.radians(lat)

lo = math.radians(lon)

la2 = math.asin(

math.sin(la) * math.cos(dist_m / R)

+ math.cos(la) * math.sin(dist_m / R) * math.cos(b)

)

lo2 = lo + math.atan2(

math.sin(b) * math.sin(dist_m / R) * math.cos(la),

math.cos(dist_m / R) - math.sin(la) * math.sin(la2),

)

return math.degrees(la2), math.degrees(lo2)

def wrap180(a):

return (a + 180) % 360 - 180

# ─────────────────────────────────────────────────────────────

#  LIGHTWEIGHT LOCAL IDENTIFICATION

#

#  No internet/model download is required for this script to run.

#  It uses local OpenCV detectors and simple scene heuristics so the

#  Pi can label what it sees and record annotated video locally.

# ─────────────────────────────────────────────────────────────

def _find_haar_dir():

candidates = [

"/usr/share/opencv4/haarcascades",

"/usr/share/opencv/haarcascades",

"/usr/local/share/opencv4/haarcascades",

]

xml = "haarcascade_frontalface_default.xml"

for d in candidates:

if os.path.isfile(os.path.join(d, xml)):

return d

for root_dir in ["/usr/share", "/usr/local/share"]:

for dp, _, files in os.walk(root_dir):

if xml in files:

return dp

return None

def _load_cascade(*names):

base = _find_haar_dir()

if base:

for name in names:

p = os.path.join(base, name)

if os.path.isfile(p):

c = cv2.CascadeClassifier(p)

if not c.empty():

return c

for root in [os.getcwd(), os.path.dirname(os.path.abspath(__file__))]:

for name in names:

p = os.path.join(root, name)

if os.path.isfile(p):

c = cv2.CascadeClassifier(p)

if not c.empty():

return c

return None

face_cascade = _load_cascade("haarcascade_frontalface_default.xml")

upperbody_cascade = _load_cascade("haarcascade_upperbody.xml")

fullbody_cascade = _load_cascade("haarcascade_fullbody.xml")

cat_cascade = _load_cascade("haarcascade_frontalcatface_extended.xml", "haarcascade_frontalcatface.xml")

car_cascade = _load_cascade("haarcascade_car.xml", "cars.xml")

PERSON_HOG = cv2.HOGDescriptor()

PERSON_HOG.setSVMDetector(cv2.HOGDescriptor_getDefaultPeopleDetector())

def _unique_labels(dets):

out = []

seen = set()

for d in dets:

lab = d["label"]

if lab not in seen:

out.append(lab)

seen.add(lab)

return out

def _detect_scene(frame_rgb):

dets = []

try:

frame_bgr = cv2.cvtColor(frame_rgb, cv2.COLOR_RGB2BGR)

gray = cv2.cvtColor(frame_rgb, cv2.COLOR_RGB2GRAY)

h, w = gray.shape

def add_boxes(cascade, label, min_size=(60, 60), scale_factor=1.1, min_neighbors=5):

if cascade is None:

return

rects = cascade.detectMultiScale(gray, scale_factor, min_neighbors, minSize=min_size)

for (x, y, bw, bh) in rects[:4]:

dets.append({"label": label, "box": (int(x), int(y), int(bw), int(bh))})

# People detector on a resized image for speed

small = cv2.resize(frame_bgr, (0, 0), fx=HOG_SCALE, fy=HOG_SCALE)

rects, _weights = PERSON_HOG.detectMultiScale(small, winStride=(8, 8), padding=(8, 8), scale=1.05)

for (x, y, bw, bh) in rects[:4]:

dets.append({

"label": "person",

"box": (int(x / HOG_SCALE), int(y / HOG_SCALE), int(bw / HOG_SCALE), int(bh / HOG_SCALE))

})

add_boxes(face_cascade, "face")

add_boxes(upperbody_cascade, "upper-body")

add_boxes(fullbody_cascade, "body")

add_boxes(cat_cascade, "animal/cat", min_size=(50, 50))

add_boxes(car_cascade, "car", min_size=(70, 40))

# Simple green-scene heuristic for trees / vegetation

hsv = cv2.cvtColor(frame_rgb, cv2.COLOR_RGB2HSV)

green_mask = cv2.inRange(hsv, (30, 35, 25), (95, 255, 255))

green_ratio = float(np.count_nonzero(green_mask)) / float(green_mask.size)

if green_ratio >= VEGETATION_GREEN_RATIO:

dets.append({"label": "tree/vegetation", "box": None})

# Very lightweight vehicle-like heuristic if no explicit car cascade exists

lower = frame_bgr[h // 2 :, :]

edges = cv2.Canny(lower, 80, 160)

cnts, _ = cv2.findContours(edges, cv2.RETR_EXTERNAL, cv2.CHAIN_APPROX_SIMPLE)

vehicle_like = False

for c in cnts[:80]:

x, y, bw, bh = cv2.boundingRect(c)

area = bw * bh

aspect = bw / float(max(1, bh))

if area > (w * h * 0.02) and 1.2 <= aspect <= 4.5 and bh < h * 0.22:

vehicle_like = True

break

if vehicle_like and car_cascade is None:

dets.append({"label": "vehicle-like", "box": None})

except Exception:

pass

labels = _unique_labels(dets)

summary = ", ".join(labels[:6]) if labels else "no major object tagged"

return dets, summary

def _draw_scene_overlay(frame_rgb, dets, summary, recording=False):

disp = frame_rgb.copy()

if SHOW_COLOR_SWATCHES:

cv2.rectangle(disp, (10, 10), (40, 35), (255, 0, 0), -1)

cv2.putText(disp, "R", (14, 31), cv2.FONT_HERSHEY_SIMPLEX, 0.6, (255, 255, 255), 2)

cv2.rectangle(disp, (50, 10), (80, 35), (0, 255, 0), -1)

cv2.putText(disp, "G", (54, 31), cv2.FONT_HERSHEY_SIMPLEX, 0.6, (0, 0, 0), 2)

cv2.rectangle(disp, (90, 10), (120, 35), (0, 0, 255), -1)

cv2.putText(disp, "B", (94, 31), cv2.FONT_HERSHEY_SIMPLEX, 0.6, (255, 255, 255), 2)

for det in dets:

box = det.get("box")

label = det["label"]

if box is None:

continue

x, y, bw, bh = box

cv2.rectangle(disp, (x, y), (x + bw, y + bh), (56, 132, 255), 2)

cv2.putText(disp, label, (x, max(22, y - 8)), cv2.FONT_HERSHEY_SIMPLEX, 0.58, (20, 20, 20), 3)

cv2.putText(disp, label, (x, max(22, y - 8)), cv2.FONT_HERSHEY_SIMPLEX, 0.58, (255, 255, 255), 1)

footer = f"Detected: {summary}"

cv2.rectangle(disp, (0, disp.shape[0] - 34), (disp.shape[1], disp.shape[0]), (245, 247, 250), -1)

cv2.putText(disp, footer[:100], (12, disp.shape[0] - 12), cv2.FONT_HERSHEY_SIMPLEX, 0.62, (50, 60, 70), 2)

if recording:

cv2.circle(disp, (disp.shape[1] - 24, 22), 8, (255, 40, 40), -1)

cv2.putText(disp, "REC", (disp.shape[1] - 68, 28), cv2.FONT_HERSHEY_SIMPLEX, 0.65, (255, 40, 40), 2)

return disp

def request_streams(m):

streams = [

(mavutil.mavlink.MAV_DATA_STREAM_EXTENDED_STATUS, 5),

(mavutil.mavlink.MAV_DATA_STREAM_EXTRA1, 20),

(mavutil.mavlink.MAV_DATA_STREAM_POSITION, 10),

(mavutil.mavlink.MAV_DATA_STREAM_RAW_SENSORS, 20),

(mavutil.mavlink.MAV_DATA_STREAM_EXTRA2, 10),

(mavutil.mavlink.MAV_DATA_STREAM_RC_CHANNELS, 20),

]

for stream, rate in streams:

m.mav.request_data_stream_send(

m.target_system, m.target_component, stream, rate, 1)

# ─────────────────────────────────────────────────────────────

#  AUTOPILOT MODES

# ─────────────────────────────────────────────────────────────

class APMode:

NONE = "none"

GPS  = "gps_waypoint"

DR   = "dead_reckoning"

class DRInputMode:

BEARING_DIST = "bearing_distance"

COORDINATES  = "coordinates"

HEADING      = "heading"

class DirectServoController:

def __init__(self, gpio_pin):

self.gpio_pin = int(gpio_pin)

self.backend = "unavailable"

self.error = ""

self._pi = None

self._gpio = None

self._pwm = None

try:

import pigpio

pi = pigpio.pi()

if pi.connected:

self._pi = pi

self.backend = "pigpio"

self._pi.set_mode(self.gpio_pin, pigpio.OUTPUT)

self._pi.set_servo_pulsewidth(self.gpio_pin, 0)

return

self.error = "pigpio daemon not running"

except Exception as ex:

self.error = str(ex)

try:

import RPi.GPIO as GPIO

GPIO.setwarnings(False)

GPIO.setmode(GPIO.BCM)

GPIO.setup(self.gpio_pin, GPIO.OUT)

self._gpio = GPIO

self._pwm = GPIO.PWM(self.gpio_pin, 50)

self._pwm.start(0)

self.backend = "RPi.GPIO"

self.error = ""

return

except Exception as ex:

if self.error:

self.error += f" | RPi.GPIO fallback failed: {ex}"

else:

self.error = f"RPi.GPIO fallback failed: {ex}"

def describe(self):

if self.backend == "unavailable":

return f"GPIO{self.gpio_pin} unavailable"

return f"GPIO{self.gpio_pin} via {self.backend}"

def set_pulsewidth(self, pulse_us):

pulse_us = int(max(500, min(2500, pulse_us)))

if self.backend == "pigpio" and self._pi is not None:

self._pi.set_servo_pulsewidth(self.gpio_pin, pulse_us)

return

if self.backend == "RPi.GPIO" and self._pwm is not None:

duty = (pulse_us / 20000.0) * 100.0

self._pwm.ChangeDutyCycle(duty)

def disable(self):

if self.backend == "pigpio" and self._pi is not None:

self._pi.set_servo_pulsewidth(self.gpio_pin, 0)

elif self.backend == "RPi.GPIO" and self._pwm is not None:

self._pwm.ChangeDutyCycle(0)

def close(self):

try:

self.disable()

except Exception:

pass

try:

if self.backend == "pigpio" and self._pi is not None:

self._pi.stop()

except Exception:

pass

try:

if self.backend == "RPi.GPIO" and self._pwm is not None:

self._pwm.stop()

if self.backend == "RPi.GPIO" and self._gpio is not None:

self._gpio.cleanup(self.gpio_pin)

except Exception:

pass

# ─────────────────────────────────────────────────────────────

#  SHARED STATE  (thread-safe via self.lock)

# ─────────────────────────────────────────────────────────────

class SharedState:

def __init__(self):

self.lock = threading.Lock()

# Keyboard stick targets

self.k_roll  = NEUTRAL

self.k_pitch = NEUTRAL

self.k_yaw   = NEUTRAL

self.manual_backoff = False

# Smoothed PWM outputs

self.o_roll  = NEUTRAL

self.o_pitch = NEUTRAL

self.o_yaw   = NEUTRAL

# Manual/mission state

self.locked     = False

self.ap_mode       = APMode.NONE

self.ap_active     = False

self.ap_status     = "Idle"

self.ap_target_lat = None

self.ap_target_lon = None

self.ap_roll_pwm   = NEUTRAL

self.ap_pitch_pwm  = NEUTRAL

self.ap_yaw_pwm    = NEUTRAL

self.force_tx_release = False

# Dead reckoning

self.dr_input_mode     = DRInputMode.BEARING_DIST

self.dr_target_bearing = 0.0

self.dr_target_dist_m  = 0.0

self.dr_remaining_m    = 0.0

self.dr_travelled_m    = 0.0

self.dr_start_lat      = None

self.dr_start_lon      = None

self.dr_est_lat        = None

self.dr_est_lon        = None

self.dr_target_lat     = None

self.dr_target_lon     = None

self.dr_phase          = "idle"

self.dr_align_since    = None

self.dr_last_update    = None

# Mission memory / auto return

self.mission_start_lat   = None

self.mission_start_lon   = None

self.auto_return_enabled = True

self.returning_to_start  = False

self.return_context      = ""

self.payload_stage       = "idle"

self.payload_stage_deadline = 0.0

# Video / identification

self.latest_detections   = []

self.detection_summary   = "no major object tagged"

self.recording           = False

self.record_path         = ""

self.record_started_at   = 0.0

self.record_error        = ""

# Servo release (direct Raspberry Pi GPIO control)

self.servo_target_pwm = SERVO_CLOSE_PWM

self.servo_status     = "CLOSED"

# Override switch (from RC_CHANNELS telemetry)

self.override_switch_pwm = 0

self.override_active     = False

# Telemetry dict

self.tel = {

"mode": "UNKNOWN", "armed": False,

"bat_v": None, "bat_a": None,

"roll_deg": None, "pitch_deg": None, "yaw_deg": None,

"heading_deg": None, "rel_alt_m": None, "alt_msl_m": None,

"gs_mps": None, "climb_mps": None,

"gps_fix": None, "gps_sats": None,

"lat": None, "lon": None,

"ax": None, "ay": None, "az": None,

"gx": None, "gy": None, "gz": None,

"thr_pwm": None, "sw_pwm": None,

}

self.latest_frame = None

self.rc_channels  = {}

self.stop = False

# ─────────────────────────────────────────────────────────────

#  CAMERA THREAD

#

#  COLOUR FIX (v11.3):

#  v11.1 forced a BGR → RGB conversion on every frame.  On this

#  user's Pi Camera Module 3 NoIR setup, that produced the exact

#  symptom being reported: blue/cyan skin tones and blue fabric

#  turning orange/red.

#

#  v11.2 therefore uses the camera's native bytes directly by

#  default and keeps the R/B swap as a simple config toggle:

#      CAMERA_SWAP_RB = False   → use frame as-is  (default)

#      CAMERA_SWAP_RB = True    → apply cv2 BGR→RGB swap

#

#  The top-left R/G/B swatches remain for quick verification.

# ─────────────────────────────────────────────────────────────

def camera_thread_fn(S: SharedState):

os.makedirs(RECORD_DIR, exist_ok=True)

picam2 = Picamera2()

cfg = picam2.create_video_configuration(

main={"size": (1280, 720), "format": CAMERA_NATIVE_FORMAT},

controls={"FrameRate": TARGET_FPS},

)

picam2.configure(cfg)

picam2.start()

writer = None

writer_path = ""

detections = []

summary = "starting…"

frame_count = 0

try:

while True:

with S.lock:

if S.stop:

break

rec_on = S.recording

raw_frame = picam2.capture_array()

if CAMERA_SWAP_RB:

frame = cv2.cvtColor(raw_frame, cv2.COLOR_BGR2RGB)

else:

frame = raw_frame.copy()

if frame_count % ID_EVERY == 0:

detections, summary = _detect_scene(frame)

disp = _draw_scene_overlay(frame, detections, summary, recording=rec_on)

if rec_on and writer is None:

ts = datetime.now().strftime("%Y%m%d_%H%M%S")

writer_path = os.path.join(RECORD_DIR, f"gcs_capture_{ts}.mp4")

fourcc = cv2.VideoWriter_fourcc(*"mp4v")

writer = cv2.VideoWriter(writer_path, fourcc, 20.0, (disp.shape[1], disp.shape[0]))

with S.lock:

S.record_path = writer_path

S.record_started_at = time.time()

if not writer.isOpened():

S.record_error = "Could not open video writer"

S.recording = False

writer.release()

writer = None

else:

S.record_error = ""

elif (not rec_on) and writer is not None:

writer.release()

writer = None

if writer is not None:

try:

writer.write(cv2.cvtColor(disp, cv2.COLOR_RGB2BGR))

except Exception:

with S.lock:

S.record_error = "Recording write failed"

S.recording = False

writer.release()

writer = None

with S.lock:

S.latest_frame = disp

S.latest_detections = [d["label"] for d in detections]

S.detection_summary = summary

S.locked = False

frame_count += 1

finally:

if writer is not None:

writer.release()

picam2.stop()

# ─────────────────────────────────────────────────────────────

#  MAVLINK WORKER  (QThread)

# ─────────────────────────────────────────────────────────────

class MavWorker(QObject):

connected = pyqtSignal(str)

error     = pyqtSignal(str)

def __init__(self, S: SharedState):

super().__init__()

self.S = S

self.m = None

self._running = True

def run(self):

try:

m  = mavutil.mavlink_connection(PORT)

hb = m.wait_heartbeat(timeout=15)

request_streams(m)

self.m = m

self.connected.emit(

f"Connected  |  sys={m.target_system}  |  {mavutil.mode_string_v10(hb)}")

except Exception as ex:

self.error.emit(str(ex))

return

while self._running:

with self.S.lock:

if self.S.stop:

break

for _ in range(250):

msg = self.m.recv_match(blocking=False)

if not msg:

break

t = msg.get_type()

with self.S.lock:

tel = self.S.tel

if t == "HEARTBEAT":

tel["mode"]  = mavutil.mode_string_v10(msg)

tel["armed"] = bool(

msg.base_mode & mavutil.mavlink.MAV_MODE_FLAG_SAFETY_ARMED)

elif t == "SYS_STATUS":

if msg.voltage_battery not in (None, 0xFFFF):

tel["bat_v"] = msg.voltage_battery / 1000

if msg.current_battery not in (None, -1):

tel["bat_a"] = msg.current_battery / 100

elif t == "ATTITUDE":

tel["roll_deg"]  = math.degrees(msg.roll)

tel["pitch_deg"] = math.degrees(msg.pitch)

tel["yaw_deg"]   = math.degrees(msg.yaw)

elif t == "VFR_HUD":

tel["heading_deg"] = msg.heading

tel["gs_mps"]      = msg.groundspeed

tel["climb_mps"]   = msg.climb

elif t == "GLOBAL_POSITION_INT":

tel["rel_alt_m"] = msg.relative_alt / 1000

tel["alt_msl_m"] = msg.alt / 1000

elif t == "GPS_RAW_INT":

tel["gps_fix"]  = msg.fix_type

tel["gps_sats"] = msg.satellites_visible

tel["lat"]      = msg.lat / 1e7

tel["lon"]      = msg.lon / 1e7

elif t == "RAW_IMU":

g = 9.80665 / 1000

tel["ax"] = msg.xacc * g

tel["ay"] = msg.yacc * g

tel["az"] = msg.zacc * g

tel["gx"] = msg.xgyro / 1000

tel["gy"] = msg.ygyro / 1000

tel["gz"] = msg.zgyro / 1000

elif t == "RC_CHANNELS":

raw3 = getattr(msg, "chan3_raw", 0)

tel["thr_pwm"] = raw3 if raw3 else None

rc_vals = {}

for ch_n in range(1, 19):

v = getattr(msg, f"chan{ch_n}_raw", 0) or 0

if v > 0:

rc_vals[ch_n] = v

self.S.rc_channels = rc_vals

sw_ch = OVERRIDE_SWITCH_CH

sw = rc_vals.get(sw_ch, 0)

tel["sw_pwm"] = sw

self.S.override_switch_pwm = sw

if sw > SWITCH_HIGH_PWM:

self.S.override_active = True

elif 0 < sw < SWITCH_LOW_PWM:

self.S.override_active = False

time.sleep(0.01)

def stop(self):

self._running = False

# ─────────────────────────────────────────────────────────────

#  ARTIFICIAL HORIZON  (custom painted widget)

# ─────────────────────────────────────────────────────────────

class HorizonWidget(QWidget):

def __init__(self, parent=None):

super().__init__(parent)

self.roll_deg  = 0.0

self.pitch_deg = 0.0

self._valid    = False

self.setMinimumSize(340, 200)

self.setSizePolicy(QSizePolicy.Expanding, QSizePolicy.Expanding)

def update_attitude(self, roll, pitch):

if roll is None or pitch is None:

self._valid = False

else:

self.roll_deg  = roll

self.pitch_deg = pitch

self._valid    = True

self.update()

def paintEvent(self, event):

p = QPainter(self)

p.setRenderHint(QPainter.Antialiasing)

w, h = self.width(), self.height()

cx, cy = w / 2, h / 2

# Background

p.fillRect(0, 0, w, h, QColor(C_CARD))

if not self._valid:

p.setPen(QColor(C_DIM))

p.setFont(QFont("Consolas", 11))

p.drawText(self.rect(), Qt.AlignCenter, "Waiting for attitude data…")

return

pitch_px = self.pitch_deg * (h / 45.0)

sky_h = int(cy - pitch_px)

# Rotated sky + ground

p.save()

p.translate(cx, cy)

p.rotate(-self.roll_deg)

p.translate(-cx, -cy)

sky = QLinearGradient(0, 0, 0, sky_h)

sky.setColorAt(0, QColor("#06101f"))

sky.setColorAt(1, QColor("#1a4a8a"))

p.fillRect(-w, -h, w * 3, sky_h + int(cy), QBrush(sky))

gnd = QLinearGradient(0, sky_h, 0, h)

gnd.setColorAt(0, QColor("#5c3d1e"))

gnd.setColorAt(1, QColor("#2a1a08"))

p.fillRect(-w, sky_h, w * 3, h * 3, QBrush(gnd))

# Horizon line

p.setPen(QPen(QColor(C_TEXT), 2))

p.drawLine(-w, int(cy - pitch_px), w * 2, int(cy - pitch_px))

# Pitch ladder

p.setPen(QPen(QColor(C_TEXT), 1))

p.setFont(QFont("Consolas", 8))

for deg in range(-30, 31, 10):

if deg == 0:

continue

y_line = int(cy - pitch_px - deg * (h / 45.0))

half = (w // 6) if deg % 20 == 0 else (w // 10)

p.drawLine(int(cx - half), y_line, int(cx + half), y_line)

p.drawText(int(cx + half + 4), y_line + 4, f"{abs(deg)}")

p.restore()

# Aircraft reference (fixed)

p.setPen(QPen(QColor(C_YELLOW), 3))

p.drawLine(int(cx - 50), int(cy), int(cx - 15), int(cy))

p.drawLine(int(cx + 15), int(cy), int(cx + 50), int(cy))

p.drawLine(int(cx), int(cy - 8), int(cx), int(cy + 8))

# Roll arc

arc_r = min(w, h) * 0.42

rect = QRectF(cx - arc_r, cy - arc_r, arc_r * 2, arc_r * 2)

p.setPen(QPen(QColor(C_ACCENT), 2))

p.drawArc(rect, int((90 - 60) * 16), int(120 * 16))

# Roll pointer

p.save()

p.translate(cx, cy)

p.rotate(-self.roll_deg)

p.setPen(QPen(QColor(C_YELLOW), 3))

p.drawLine(0, int(-arc_r + 2), 0, int(-arc_r + 14))

p.restore()

# Text overlay

p.setPen(QColor(C_TEXT))

p.setFont(QFont("Consolas", 10, QFont.Bold))

p.drawText(8, 16, f"Roll  {self.roll_deg:+.1f}°")

p.drawText(8, 32, f"Pitch {self.pitch_deg:+.1f}°")

# ─────────────────────────────────────────────────────────────

#  MAP PAGE  (intercepts gcs:// URLs from Leaflet clicks)

# ─────────────────────────────────────────────────────────────

class MapPage(QWebEnginePage):

waypoint_set = pyqtSignal(float, float)

def acceptNavigationRequest(self, url, nav_type, is_main):

s = url.toString()

if s.startswith("gcs://wp/"):

try:

_, _, coords = s.partition("gcs://wp/")

lat_s, lon_s = coords.split("/")

self.waypoint_set.emit(float(lat_s), float(lon_s))

except Exception:

pass

return False

return super().acceptNavigationRequest(url, nav_type, is_main)

# ─────────────────────────────────────────────────────────────

#  LEAFLET MAP HTML

# ─────────────────────────────────────────────────────────────

LEAFLET_HTML = """<!DOCTYPE html>

<html><head><meta charset="utf-8"/>

<style>

body,html{{margin:0;padding:0;background:#0b0f14;}}

#map{{width:100vw;height:100vh;}}

@keyframes pulsering{{

0%  {{transform:scale(1);  opacity:0.8;}}

60% {{transform:scale(1.7);opacity:0.0;}}

100%{{transform:scale(1);  opacity:0.0;}}

}}

</style>

<link rel="stylesheet" href="https://unpkg.com/leaflet@1.9.4/dist/leaflet.css"/>

<script src="https://unpkg.com/leaflet@1.9.4/dist/leaflet.js"></script>

</head><body><div id="map"></div>

<script>

var map=L.map('map',{{zoomControl:true}}).setView([{LAT},{LON}],{ZOOM});

L.tileLayer('https://{{s}}.tile.openstreetmap.org/{{z}}/{{x}}/{{y}}.png',{{

attribution:'&copy; OSM',maxZoom:19}}).addTo(map);

map.getContainer().style.filter='brightness(1.05) saturate(0.95)';

function makeDroneIcon(h){{

h=h||0;

var svg='<svg xmlns="http://www.w3.org/2000/svg" width="56" height="56" viewBox="-28 -28 56 56">'+

'<circle cx="0" cy="0" r="26" fill="rgba(77,166,255,0.12)" stroke="#4da6ff" stroke-width="1.5"/>'+

'<g transform="rotate('+h+')">'+

'<polygon points="0,-22 9,6 0,0 -9,6" fill="#4da6ff" stroke="#fff" stroke-width="1.8" stroke-linejoin="round"/>'+

'<polygon points="0,0 5,14 0,10 -5,14" fill="#2563eb" stroke="#fff" stroke-width="1"/>'+

'</g>'+

'<circle cx="0" cy="0" r="4" fill="#fff" stroke="#4da6ff" stroke-width="2"/>'+

'</svg>'+

'<div style="position:absolute;left:50%;top:58px;transform:translateX(-50%);">'+

'<span style="background:#2563eb;color:#fff;font-family:monospace;font-size:10px;'+

'font-weight:bold;padding:2px 6px;border-radius:3px;white-space:nowrap;'+

'box-shadow:0 1px 4px rgba(0,0,0,0.5);">'+Math.round(h)+'\\u00b0</span></div>';

return L.divIcon({{

html:'<div style="position:relative;width:56px;height:56px;">'+svg+'</div>',

className:'',iconSize:[56,56],iconAnchor:[28,28],popupAnchor:[0,-30]}});

}}

function makeWpIcon(){{

var html='<div style="position:relative;width:60px;height:60px;">'+

'<div style="position:absolute;width:56px;height:56px;border-radius:50%;'+

'border:3px solid #e3b341;top:2px;left:2px;animation:pulsering 1.8s ease-out infinite;"></div>'+

'<div style="position:absolute;width:42px;height:42px;border-radius:50%;'+

'border:3px solid #e3b341;background:rgba(227,179,65,0.12);top:9px;left:9px;"></div>'+

'<div style="position:absolute;width:2px;height:42px;background:#e3b341;top:9px;left:29px;"></div>'+

'<div style="position:absolute;width:42px;height:2px;background:#e3b341;top:29px;left:9px;"></div>'+

'<div style="position:absolute;width:10px;height:10px;border-radius:50%;'+

'background:#e3b341;border:2px solid #fff;top:25px;left:25px;"></div>'+

'<div style="position:absolute;left:50%;top:62px;transform:translateX(-50%);">'+

'<span style="background:#e3b341;color:#000;font-family:monospace;font-size:10px;'+

'font-weight:bold;padding:2px 6px;border-radius:3px;white-space:nowrap;'+

'box-shadow:0 1px 4px rgba(0,0,0,0.5);">TARGET</span></div></div>';

return L.divIcon({{html:html,className:'',iconSize:[60,60],iconAnchor:[30,30],popupAnchor:[0,-35]}});

}}

var droneMarker=null,wpMarker=null;

function setDrone(lat,lon,heading){{

var icon=makeDroneIcon(heading||0);

if(!droneMarker){{droneMarker=L.marker([lat,lon],{{icon:icon,zIndexOffset:1000}}).addTo(map);}}

else{{droneMarker.setLatLng([lat,lon]);droneMarker.setIcon(icon);}}

}}

function setWaypoint(lat,lon){{

var icon=makeWpIcon();

if(!wpMarker){{wpMarker=L.marker([lat,lon],{{icon:icon,zIndexOffset:500}}).addTo(map);}}

else{{wpMarker.setLatLng([lat,lon]);}}

}}

function clearWaypoint(){{if(wpMarker){{map.removeLayer(wpMarker);wpMarker=null;}}}}

map.on('click',function(e){{

window.location.href='gcs://wp/'+e.latlng.lat.toFixed(7)+'/'+e.latlng.lng.toFixed(7);

}});

</script></body></html>

""".format(LAT=DEFAULT_MAP_LAT, LON=DEFAULT_MAP_LON, ZOOM=DEFAULT_MAP_ZOOM)

# ─────────────────────────────────────────────────────────────

#  STYLESHEET

# ─────────────────────────────────────────────────────────────

QSS = f"""

/* ── Base ───────────────────────────────────────────── */

QMainWindow, QWidget {{

background: {C_BG};

color: {C_TEXT};

font-family: "Segoe UI", "Ubuntu", "Cantarell", sans-serif;

font-size: 13px;

}}

/* ── Tabs ───────────────────────────────────────────── */

QTabWidget::pane {{

border: 1px solid {C_BORDER};

background: {C_BG};

border-radius: 8px;

}}

QTabBar::tab {{

background: {C_SURFACE};

color: {C_DIM};

border: 1px solid {C_BORDER};

padding: 9px 24px;

margin-right: 2px;

border-top-left-radius: 8px;

border-top-right-radius: 8px;

font-weight: 600;

}}

QTabBar::tab:selected {{

background: {C_BG};

color: {C_TEXT};

border-bottom: 2px solid {C_ACCENT};

}}

QTabBar::tab:hover {{

color: {C_TEXT};

background: {C_CARD};

}}

/* ── Cards (QGroupBox) ──────────────────────────────── */

QGroupBox {{

border: 1px solid {C_BORDER};

border-radius: 10px;

margin-top: 12px;

padding: 14px 10px 10px 10px;

background: {C_CARD};

}}

QGroupBox::title {{

subcontrol-origin: margin;

subcontrol-position: top left;

padding: 0 10px;

color: {C_DIM};

font-weight: bold;

font-size: 11px;

letter-spacing: 0.8px;

text-transform: uppercase;

}}

/* ── Buttons ────────────────────────────────────────── */

QPushButton {{

background: {C_SURFACE};

color: {C_TEXT};

border: 1px solid {C_BORDER};

border-radius: 8px;

padding: 8px 16px;

font-size: 13px;

font-weight: 500;

}}

QPushButton:hover {{

background: {C_CARD};

border-color: {C_ACCENT};

}}

QPushButton:pressed {{

background: {C_BG};

}}

QPushButton#btnFly {{

background: {C_ACCENT};

color: #000;

border: none;

font-weight: bold;

font-size: 14px;

padding: 10px;

border-radius: 8px;

}}

QPushButton#btnFly:hover {{

background: #79c0ff;

}}

QPushButton#btnAbort {{

background: {C_RED};

color: white;

border: none;

font-weight: bold;

font-size: 14px;

padding: 10px;

border-radius: 8px;

}}

QPushButton#btnAbort:hover {{

background: #ff8080;

}}

QPushButton#btnUnlock {{

background: {C_GREEN};

color: #000;

border: none;

font-weight: bold;

font-size: 13px;

padding: 8px 16px;

border-radius: 8px;

}}

QPushButton#btnUnlock:hover {{

background: #5fe07a;

}}

/* ── Inputs ─────────────────────────────────────────── */

QLineEdit, QSpinBox {{

background: {C_BG};

color: {C_TEXT};

border: 1px solid {C_BORDER};

border-radius: 6px;

padding: 6px 10px;

font-family: "Consolas", "Courier New", monospace;

}}

QLineEdit:focus, QSpinBox:focus {{

border-color: {C_ACCENT};

}}

QSpinBox::up-button, QSpinBox::down-button {{

background: {C_SURFACE};

border: 1px solid {C_BORDER};

}}

"""

# ─────────────────────────────────────────────────────────────

#  UI HELPERS

# ─────────────────────────────────────────────────────────────

def card(title):

return QGroupBox(title)

def mono(text="—", color=None, size=12):

lbl = QLabel(text)

style = f"font-family: Consolas, monospace; font-size: {size}px;"

if color:

style += f" color: {color};"

lbl.setStyleSheet(style)

return lbl

def _sep():

s = QFrame()

s.setFrameShape(QFrame.VLine)

s.setStyleSheet(f"color: {C_BORDER};")

return s

# ─────────────────────────────────────────────────────────────

#  MAIN WINDOW

# ─────────────────────────────────────────────────────────────

class GCSWindow(QMainWindow):

def __init__(self, S: SharedState):

super().__init__()

self.S = S

self.m = None

self._wp_lat = None

self._wp_lon = None

self._B_bearing = None

self._B_dist = None

self._mav_thread = None

self._mav_worker = None

self._release_count = 0

self._last_servo_pwm_sent = None

self.servo_hw = DirectServoController(SERVO_GPIO_PIN)

self.setWindowTitle("Medical Delivery Drone GCS v50 Light  |  Pi local go-to + direct GPIO servo")

self.resize(1480, 900)

self.setStyleSheet(QSS)

self.setFocusPolicy(Qt.StrongFocus)

self._build_ui()

self._connect_mavlink()

self._start_timers()

self._apply_servo_pwm_direct(SERVO_CLOSE_PWM)

# ── MAVLink connection ────────────────────────────────────

def _connect_mavlink(self):

self._mav_thread = QThread()

self._mav_worker = MavWorker(self.S)

self._mav_worker.moveToThread(self._mav_thread)

self._mav_thread.started.connect(self._mav_worker.run)

self._mav_worker.connected.connect(self._on_mav_connected)

self._mav_worker.error.connect(self._on_mav_error)

self._mav_thread.start()

def _on_mav_connected(self, msg):

self.m = self._mav_worker.m

self.status_lbl.setText(f"  {msg}")

self.status_lbl.setStyleSheet(

f"color: {C_GREEN}; font-weight: bold; font-size: 12px;")

def _on_mav_error(self, msg):

self.status_lbl.setText(f"  Connection failed: {msg}")

self.status_lbl.setStyleSheet(

f"color: {C_RED}; font-weight: bold; font-size: 12px;")

# ── Timers ────────────────────────────────────────────────

def _start_timers(self):

self._t_ui = QTimer(self)

self._t_ui.timeout.connect(self._tick_ui)

self._t_ui.start(int(1000 / UI_HZ))

self._t_send = QTimer(self)

self._t_send.timeout.connect(self._tick_send)

self._t_send.start(int(1000 / SEND_HZ))

self._t_ap = QTimer(self)

self._t_ap.timeout.connect(self._tick_ap)

self._t_ap.start(100)

self._t_map = QTimer(self)

self._t_map.timeout.connect(self._tick_map)

self._t_map.start(500)

self._t_servo = QTimer(self)

self._t_servo.timeout.connect(self._tick_servo)

self._t_servo.start(50)

# ══════════════════════════════════════════════════════════

#  BUILD UI

# ══════════════════════════════════════════════════════════

def _build_ui(self):

central = QWidget()

self.setCentralWidget(central)

root = QVBoxLayout(central)

root.setContentsMargins(12, 8, 12, 8)

root.setSpacing(6)

# ── Header ────────────────────────────────────────────

hdr = QHBoxLayout()

title = QLabel("MEDICAL DELIVERY DRONE GCS")

title.setStyleSheet(

f"font-size: 16px; font-weight: bold; color: {C_ACCENT};"

f" letter-spacing: 2px;")

ver = QLabel("v50 Light")

ver.setStyleSheet(f"color: {C_DIM}; font-size: 12px;")

self.status_lbl = QLabel("  Connecting to Pixhawk…")

self.status_lbl.setStyleSheet(f"color: {C_YELLOW}; font-size: 12px;")

hdr.addWidget(title)

hdr.addWidget(ver)

hdr.addStretch()

hdr.addWidget(self.status_lbl)

root.addLayout(hdr)

# ── Status strip ──────────────────────────────────────

self._build_status_strip()

root.addWidget(self._strip_frame)

# ── Tabs ──────────────────────────────────────────────

self.tabs = QTabWidget()

root.addWidget(self.tabs, stretch=1)

self._build_dashboard_tab()

self._build_map_tab()

self._build_nav_tab()

# ── Keyboard hint ─────────────────────────────────────

hint = QLabel(

"W/S pitch  ·  A/D roll  ·  ←/→ yaw  ·  SPACE center sticks  ·  8 servo mid  ·  9 servo open  ·  0 servo close  ·  X release  ·  R record  ·  ESC quit  ·  "

f"TX ch{OVERRIDE_SWITCH_CH} HIGH → Pi local stick simulation")

hint.setStyleSheet(f"color: {C_DIM}; font-size: 11px;")

hint.setAlignment(Qt.AlignCenter)

root.addWidget(hint)

# ── Status strip (compact top bar) ────────────────────────

def _build_status_strip(self):

self._strip_frame = QFrame()

self._strip_frame.setFixedHeight(32)

self._strip_frame.setStyleSheet(

f"background: {C_SURFACE}; border: 1px solid {C_BORDER};"

f" border-radius: 6px;")

row = QHBoxLayout(self._strip_frame)

row.setContentsMargins(10, 0, 10, 0)

row.setSpacing(0)

def chip(key, default, w=120):

lbl = QLabel(default)

lbl.setFixedWidth(w)

lbl.setStyleSheet(

f"font-family: Consolas, monospace; font-size: 11px; color: {C_TEXT};")

lbl.setAlignment(Qt.AlignCenter)

row.addWidget(lbl)

row.addWidget(_sep())

return lbl

self._mb = {}

self._mb["mode"]  = chip("mode",  "MODE: —",      110)

self._mb["armed"] = chip("armed", "DISARMED",      100)

self._mb["sw"]    = chip("sw",    "SW: —",         120)

self._mb["bat"]   = chip("bat",   "BAT: —V",       90)

self._mb["hdg"]   = chip("hdg",   "HDG: —°",       80)

self._mb["alt"]   = chip("alt",   "ALT: —m",       80)

self._mb["gs"]    = chip("gs",    "GS: — m/s",    100)

self._mb["gps"]   = chip("gps",   "GPS: —",       100)

self._mb["ap"]    = chip("ap",    "AP: Idle",      260)

row.addStretch()

# ── Dashboard tab ─────────────────────────────────────────

def _build_dashboard_tab(self):

tab = QWidget()

self.tabs.addTab(tab, "  Dashboard")

layout = QHBoxLayout(tab)

layout.setContentsMargins(8, 8, 8, 8)

layout.setSpacing(10)

left = QVBoxLayout()

left.setSpacing(8)

cam_box = card("Camera · Recording · Identification")

cam_lay = QVBoxLayout(cam_box)

cam_lay.setContentsMargins(4, 4, 4, 4)

self.cam_label = QLabel("Camera initialising…")

self.cam_label.setAlignment(Qt.AlignCenter)

self.cam_label.setSizePolicy(QSizePolicy.Expanding, QSizePolicy.Expanding)

self.cam_label.setStyleSheet(

f"background: #eef2f7; color: {C_DIM}; border-radius: 6px;")

cam_lay.addWidget(self.cam_label)

left.addWidget(cam_box, stretch=5)

lower = QHBoxLayout()

lower.setSpacing(8)

att_box = card("Attitude")

att_lay = QVBoxLayout(att_box)

self.horizon = HorizonWidget()

self.horizon.setMinimumHeight(160)

att_lay.addWidget(self.horizon)

lower.addWidget(att_box, stretch=1)

tel_box = card("Telemetry · RC")

tg = QGridLayout(tel_box)

tg.setSpacing(5)

tg.setContentsMargins(10, 10, 10, 10)

def trow(r, label_text, color=C_TEXT):

lbl_key = QLabel(label_text)

lbl_key.setStyleSheet(f"color: {C_DIM}; font-size: 12px;")

tg.addWidget(lbl_key, r, 0)

lbl_val = mono("—", color)

tg.addWidget(lbl_val, r, 1)

return lbl_val

self._tl = {}

self._tl["mode"]  = trow(0, "Mode")

self._tl["armed"] = trow(1, "Armed")

self._tl["bat"]   = trow(2, "Battery")

self._tl["nav"]   = trow(3, "Hdg / Alt / GS")

self._tl["gps"]   = trow(4, "GPS")

self._tl["imu"]   = trow(5, "IMU Accel")

self._tl["thr"]   = trow(6, "TX Throttle")

self._tl["rc"]    = trow(7, "RC Channels")

lower.addWidget(tel_box, stretch=1)

left.addLayout(lower, stretch=3)

layout.addLayout(left, stretch=4)

right = QVBoxLayout()

right.setSpacing(8)

sw_box = card(f"Override Switch · TX ch{OVERRIDE_SWITCH_CH}")

swl = QVBoxLayout(sw_box)

self.lbl_switch = QLabel("TX FULL CONTROL")

self.lbl_switch.setStyleSheet(

f"font-size: 18px; font-weight: bold; color: {C_GREEN};")

self.lbl_switch.setAlignment(Qt.AlignCenter)

self.lbl_switch_pwm = mono(

f"ch{OVERRIDE_SWITCH_CH} PWM: —  |  LOW<{SWITCH_LOW_PWM}  HIGH>{SWITCH_HIGH_PWM}", C_DIM)

self.lbl_switch_pwm.setAlignment(Qt.AlignCenter)

sw_cfg = QHBoxLayout()

sw_cfg.addWidget(QLabel("Switch ch:"))

self.spin_sw_ch = QSpinBox()

self.spin_sw_ch.setRange(1, 18)

self.spin_sw_ch.setValue(OVERRIDE_SWITCH_CH)

self.spin_sw_ch.setFixedWidth(70)

self.spin_sw_ch.valueChanged.connect(self._on_sw_ch_change)

sw_cfg.addWidget(self.spin_sw_ch)

sw_cfg.addStretch()

swl.addWidget(self.lbl_switch)

swl.addWidget(self.lbl_switch_pwm)

swl.addLayout(sw_cfg)

right.addWidget(sw_box)

rec_box = card("Recording · Scene Tags")

rl = QVBoxLayout(rec_box)

self.lbl_state = QLabel("MANUAL READY")

self.lbl_state.setStyleSheet(

f"font-size: 16px; font-weight: bold; color: {C_ACCENT};")

self.lbl_state.setAlignment(Qt.AlignCenter)

self.lbl_scene = mono("Detected: —", C_DIM)

self.lbl_scene.setAlignment(Qt.AlignCenter)

self.lbl_record = mono("Recording: OFF", C_DIM)

self.lbl_record.setAlignment(Qt.AlignCenter)

self.btn_record = QPushButton("START RECORDING")

self.btn_record.clicked.connect(self.toggle_recording)

rl.addWidget(self.lbl_state)

rl.addWidget(self.lbl_scene)

rl.addWidget(self.lbl_record)

rl.addWidget(self.btn_record)

right.addWidget(rec_box)

pwm_box = card("PWM Outputs · ch1/ch2/ch4")

pl = QVBoxLayout(pwm_box)

self.lbl_targets = mono("Target   R=1500  P=1500  Y=1500", C_DIM)

self.lbl_outputs = mono("Output   R=1500  P=1500  Y=1500", C_ACCENT, 13)

self.lbl_outputs.setStyleSheet(

f"font-size: 13px; font-weight: bold; color: {C_ACCENT}; font-family: Consolas;")

note = QLabel(f"ch3 throttle = NO_OVERRIDE  ·  TX owns flight mode  ·  servo on Pi GPIO{SERVO_GPIO_PIN}")

note.setStyleSheet(f"color: {C_DIM}; font-size: 10px;")

pl.addWidget(self.lbl_targets)

pl.addWidget(self.lbl_outputs)

pl.addWidget(note)

right.addWidget(pwm_box)

servo_box = card(f"Servo Release · Pi GPIO{SERVO_GPIO_PIN}")

svl = QVBoxLayout(servo_box)

self.lbl_servo = mono("Servo: CLOSED", C_ORANGE, 12)

self.lbl_servo.setAlignment(Qt.AlignCenter)

servo_btns = QHBoxLayout()

btn_mid = QPushButton("MID")

btn_mid.clicked.connect(self.set_servo_mid)

btn_open = QPushButton("OPEN")

btn_open.clicked.connect(self.set_servo_open)

btn_close = QPushButton("CLOSE")

btn_close.clicked.connect(self.set_servo_close)

servo_btns.addWidget(btn_mid)

servo_btns.addWidget(btn_open)

servo_btns.addWidget(btn_close)

svl.addWidget(self.lbl_servo)

self.lbl_servo_hw = QLabel(self.servo_hw.describe())

self.lbl_servo_hw.setStyleSheet(f"color: {C_DIM}; font-size: 11px;")

self.lbl_servo_hw.setAlignment(Qt.AlignCenter)

svl.addWidget(self.lbl_servo_hw)

svl.addLayout(servo_btns)

svl.addWidget(QLabel("Keyboard: 8 = mid  ·  9 = open  ·  0 = close"))

right.addWidget(servo_box)

ap_box = card("Mission Status")

al = QVBoxLayout(ap_box)

al.setSpacing(8)

self.lbl_ap_status = QLabel("Idle")

self.lbl_ap_status.setStyleSheet(

f"color: {C_CYAN}; font-size: 12px; font-family: Consolas;")

self.lbl_ap_status.setWordWrap(True)

al.addWidget(self.lbl_ap_status)

self.btn_abort_dash = QPushButton("ABORT CURRENT GO-TO")

self.btn_abort_dash.setObjectName("btnAbort")

self.btn_abort_dash.clicked.connect(self.do_abort)

al.addWidget(self.btn_abort_dash)

right.addWidget(ap_box)

right.addStretch()

layout.addLayout(right, stretch=2)

# ── Map tab ───────────────────────────────────────────────

def _build_map_tab(self):

tab = QWidget()

self.tabs.addTab(tab, "  Map & GPS Autopilot")

layout = QVBoxLayout(tab)

layout.setContentsMargins(8, 8, 8, 8)

layout.setSpacing(6)

gps_box = card("GPS Target · local stick-sim go-to · auto drop + auto return")

gr = QHBoxLayout(gps_box)

gr.setSpacing(10)

gr.addWidget(QLabel("Lat:"))

self.inp_gps_lat = QLineEdit("— click map —")

self.inp_gps_lat.setFixedWidth(140)

gr.addWidget(self.inp_gps_lat)

gr.addWidget(QLabel("Lon:"))

self.inp_gps_lon = QLineEdit("—")

self.inp_gps_lon.setFixedWidth(140)

gr.addWidget(self.inp_gps_lon)

self.lbl_map_dist = QLabel("Dist: —")

self.lbl_map_dist.setStyleSheet(f"color: {C_YELLOW}; font-weight: bold;")

gr.addWidget(self.lbl_map_dist)

gr.addWidget(QLabel("Payload:"))

btn_mid = QPushButton("MID")

btn_mid.clicked.connect(self.set_servo_mid)

btn_open = QPushButton("OPEN")

btn_open.clicked.connect(self.set_servo_open)

btn_close = QPushButton("CLOSE")

btn_close.clicked.connect(self.set_servo_close)

gr.addWidget(btn_mid)

gr.addWidget(btn_open)

gr.addWidget(btn_close)

gr.addStretch()

btn_fly = QPushButton("FLY TO TARGET")

btn_fly.setObjectName("btnFly")

btn_fly.setFixedWidth(180)

btn_fly.clicked.connect(self.start_gps_autopilot)

gr.addWidget(btn_fly)

btn_abort = QPushButton("ABORT")

btn_abort.setObjectName("btnAbort")

btn_abort.setFixedWidth(110)

btn_abort.clicked.connect(self.do_abort)

gr.addWidget(btn_abort)

btn_clr = QPushButton("Clear")

btn_clr.clicked.connect(self._clear_wp)

gr.addWidget(btn_clr)

layout.addWidget(gps_box)

self.map_page = MapPage()

self.map_page.waypoint_set.connect(self._on_map_click)

self.map_view = QWebEngineView()

self.map_view.setPage(self.map_page)

self.map_view.setHtml(LEAFLET_HTML, QUrl("http://localhost/"))

self.map_view.setStyleSheet("border-radius: 8px;")

layout.addWidget(self.map_view, stretch=1)

# ── Dead reckoning tab ───────────────────────────────────

def _build_nav_tab(self):

tab = QWidget()

self.tabs.addTab(tab, "  Dead Reckoning")

layout = QVBoxLayout(tab)

layout.setContentsMargins(8, 8, 8, 8)

layout.setSpacing(8)

mode_box = card("Dead Reckoning Input Mode")

ml = QHBoxLayout(mode_box)

ml.setSpacing(18)

self._dr_mode_group = QButtonGroup(self)

for i, (text_label, mode_value) in enumerate([

("Mode A — Bearing + Distance", DRInputMode.BEARING_DIST),

("Mode B — Start + End Coordinates", DRInputMode.COORDINATES),

]):

rb = QRadioButton(text_label)

rb.setProperty("drmode", mode_value)

if i == 0:

rb.setChecked(True)

self._dr_mode_group.addButton(rb, i)

ml.addWidget(rb)

ml.addStretch()

self._dr_mode_group.buttonClicked.connect(self._on_dr_mode_change)

layout.addWidget(mode_box)

self.dr_panel_area = QWidget()

self.dr_panel_layout = QVBoxLayout(self.dr_panel_area)

self.dr_panel_layout.setContentsMargins(0, 0, 0, 0)

self.dr_panel_layout.setSpacing(8)

layout.addWidget(self.dr_panel_area)

self._build_dr_panel_A()

self._build_dr_panel_B()

self._show_dr_panel(DRInputMode.BEARING_DIST)

status_box = card("Dead Reckoning Status")

stl = QVBoxLayout(status_box)

self.lbl_dr_status = QLabel("Status: Idle")

self.lbl_dr_status.setStyleSheet(

f"color: {C_CYAN}; font-size: 12px; font-family: Consolas;")

self.lbl_dr_prog = mono("0.0 / 0.0 m", C_TEXT)

self.lbl_dr_speed = mono("Speed: —", C_YELLOW)

prog_row = QHBoxLayout()

prog_row.addWidget(self.lbl_dr_prog)

prog_row.addStretch()

prog_row.addWidget(self.lbl_dr_speed)

self.dr_pbar = QProgressBar()

self.dr_pbar.setRange(0, 100)

self.dr_pbar.setValue(0)

self.dr_pbar.setTextVisible(False)

self.lbl_dr_pos = mono("Estimated position: —", C_GREEN)

payload_row = QHBoxLayout()

payload_row.addWidget(QLabel("Payload:"))

btn_mid = QPushButton("MID")

btn_mid.clicked.connect(self.set_servo_mid)

btn_open = QPushButton("OPEN")

btn_open.clicked.connect(self.set_servo_open)

btn_close = QPushButton("CLOSE")

btn_close.clicked.connect(self.set_servo_close)

payload_row.addWidget(btn_mid)

payload_row.addWidget(btn_open)

payload_row.addWidget(btn_close)

payload_row.addStretch()

btns = QHBoxLayout()

self.btn_dr_fly = QPushButton("START DEAD RECKONING")

self.btn_dr_fly.setObjectName("btnFly")

self.btn_dr_fly.clicked.connect(self.start_dr_autopilot)

self.btn_dr_abort = QPushButton("ABORT")

self.btn_dr_abort.setObjectName("btnAbort")

self.btn_dr_abort.clicked.connect(self.do_abort)

btns.addWidget(self.btn_dr_fly, stretch=3)

btns.addWidget(self.btn_dr_abort, stretch=1)

stl.addWidget(self.lbl_dr_status)

stl.addLayout(prog_row)

stl.addWidget(self.dr_pbar)

stl.addWidget(self.lbl_dr_pos)

stl.addLayout(payload_row)

stl.addLayout(btns)

layout.addWidget(status_box)

layout.addStretch()

def _build_dr_panel_A(self):

self._panel_A = card("Mode A — Bearing + Distance")

ag = QGridLayout(self._panel_A)

ag.addWidget(QLabel("Bearing (°):"), 0, 0)

self.dr_A_bearing = QLineEdit("0")

self.dr_A_bearing.setFixedWidth(90)

ag.addWidget(self.dr_A_bearing, 0, 1)

ag.addWidget(QLabel("Distance (m):"), 1, 0)

self.dr_A_dist = QLineEdit("10")

self.dr_A_dist.setFixedWidth(90)

ag.addWidget(self.dr_A_dist, 1, 1)

self.lbl_A_est = mono("", C_YELLOW, 11)

ag.addWidget(self.lbl_A_est, 2, 0, 1, 4)

self.dr_A_dist.textChanged.connect(self._refresh_A)

self._refresh_A()

self.dr_panel_layout.addWidget(self._panel_A)

def _build_dr_panel_B(self):

self._panel_B = card("Mode B — Start + End Coordinates")

bg = QGridLayout(self._panel_B)

bg.addWidget(QLabel("Start Lat:"), 0, 0)

self.dr_B_slat = QLineEdit()

self.dr_B_slat.setFixedWidth(140)

bg.addWidget(self.dr_B_slat, 0, 1)

bg.addWidget(QLabel("Start Lon:"), 0, 2)

self.dr_B_slon = QLineEdit()

self.dr_B_slon.setFixedWidth(140)

bg.addWidget(self.dr_B_slon, 0, 3)

btn_gps = QPushButton("Use GPS")

btn_gps.clicked.connect(self._use_gps_start)

bg.addWidget(btn_gps, 0, 4)

bg.addWidget(QLabel("End Lat:"), 1, 0)

self.dr_B_elat = QLineEdit()

self.dr_B_elat.setFixedWidth(140)

bg.addWidget(self.dr_B_elat, 1, 1)

bg.addWidget(QLabel("End Lon:"), 1, 2)

self.dr_B_elon = QLineEdit()

self.dr_B_elon.setFixedWidth(140)

bg.addWidget(self.dr_B_elon, 1, 3)

btn_map = QPushButton("Use Map Target")

btn_map.clicked.connect(self._use_map_end)

bg.addWidget(btn_map, 1, 4)

self.lbl_B_result = mono("Bearing: —   Distance: —", C_YELLOW, 11)

bg.addWidget(self.lbl_B_result, 2, 0, 1, 4)

btn_calc = QPushButton("Calculate")

btn_calc.clicked.connect(self._calc_B)

bg.addWidget(btn_calc, 2, 4)

self.dr_panel_layout.addWidget(self._panel_B)

def _show_dr_panel(self, mode):

for panel, mode_value in [

(self._panel_A, DRInputMode.BEARING_DIST),

(self._panel_B, DRInputMode.COORDINATES),

]:

panel.setVisible(mode_value == mode)

def _on_dr_mode_change(self, btn):

self._show_dr_panel(btn.property("drmode"))

def _current_dr_mode(self):

btn = self._dr_mode_group.checkedButton()

return btn.property("drmode") if btn else DRInputMode.BEARING_DIST

def _refresh_A(self):

try:

d = float(self.dr_A_dist.text())

self.lbl_A_est.setText(

f"Estimated travel time: ~{d / CRUISE_SPEED_MPS:.1f} s")

except Exception:

self.lbl_A_est.setText("")

def _use_gps_start(self):

with self.S.lock:

lat = self.S.tel["lat"]

lon = self.S.tel["lon"]

if lat is None or lon is None or lat == 0.0:

QMessageBox.warning(self, "No GPS", "GPS start position is not available yet.")

return

self.dr_B_slat.setText(f"{lat:.7f}")

self.dr_B_slon.setText(f"{lon:.7f}")

def _use_map_end(self):

if self._wp_lat is None or self._wp_lon is None:

QMessageBox.warning(self, "No Map Target", "Click the map first.")

return

self.dr_B_elat.setText(f"{self._wp_lat:.7f}")

self.dr_B_elon.setText(f"{self._wp_lon:.7f}")

def _calc_B(self):

try:

slat = float(self.dr_B_slat.text())

slon = float(self.dr_B_slon.text())

elat = float(self.dr_B_elat.text())

elon = float(self.dr_B_elon.text())

except ValueError:

QMessageBox.warning(self, "Input Error", "Enter valid start/end coordinates.")

return

d = haversine_m(slat, slon, elat, elon)

b = bearing_to(slat, slon, elat, elon)

self._B_bearing = b

self._B_dist = d

self.lbl_B_result.setText(

f"Bearing: {b:.1f}°   Distance: {d:.1f} m   Est: ~{d / CRUISE_SPEED_MPS:.1f} s")

# ══════════════════════════════════════════════════════════

#  KEYBOARD INPUT

# ══════════════════════════════════════════════════════════

def keyPressEvent(self, e):

k = e.key()

if k == Qt.Key_Escape:

self.close()

return

if k == Qt.Key_X:

self.do_release_overrides()

return

if k == Qt.Key_R:

self.toggle_recording()

return

if k == Qt.Key_Space:

with self.S.lock:

if not self.S.ap_active:

self.S.k_roll = self.S.k_pitch = self.S.k_yaw = NEUTRAL

self.S.o_roll = self.S.o_pitch = self.S.o_yaw = NEUTRAL

self.S.manual_backoff = False

return

if k == Qt.Key_8:

self.set_servo_mid()

return

if k == Qt.Key_9:

self.set_servo_open()

return

if k == Qt.Key_0:

self.set_servo_close()

return

with self.S.lock:

if self.S.ap_active:

return

if   k == Qt.Key_W:     self.S.k_pitch = clamp(self.S.k_pitch + STEP)

elif k == Qt.Key_S:     self.S.k_pitch = clamp(self.S.k_pitch - STEP)

elif k == Qt.Key_A:     self.S.k_roll  = clamp(self.S.k_roll  - STEP)

elif k == Qt.Key_D:     self.S.k_roll  = clamp(self.S.k_roll  + STEP)

elif k == Qt.Key_Left:  self.S.k_yaw   = clamp(self.S.k_yaw   - STEP)

elif k == Qt.Key_Right: self.S.k_yaw   = clamp(self.S.k_yaw   + STEP)

# ══════════════════════════════════════════════════════════

#  CONTROL ACTIONS

# ══════════════════════════════════════════════════════════

def try_unlock(self):

with self.S.lock:

self.S.locked = False

self.S.manual_backoff = False

self.S.force_tx_release = False

self.S.k_roll = self.S.k_pitch = self.S.k_yaw = NEUTRAL

def do_backoff(self):

with self.S.lock:

self.S.manual_backoff = False

def toggle_recording(self):

with self.S.lock:

self.S.recording = not self.S.recording

if not self.S.recording:

self.S.record_started_at = 0.0

def _set_servo_target(self, pwm, label):

with self.S.lock:

self.S.servo_target_pwm = int(pwm)

self.S.servo_status = label

self._last_servo_pwm_sent = None

def set_servo_open(self):

self._set_servo_target(SERVO_OPEN_PWM, "OPEN")

def set_servo_mid(self):

self._set_servo_target(SERVO_MID_PWM, "MID")

def set_servo_close(self):

self._set_servo_target(SERVO_CLOSE_PWM, "CLOSED")

def _apply_servo_pwm_direct(self, pwm):

try:

self.servo_hw.set_pulsewidth(int(pwm))

self._last_servo_pwm_sent = int(pwm)

except Exception as ex:

with self.S.lock:

self.S.record_error = f"Servo GPIO error: {ex}"

def _tick_servo(self):

with self.S.lock:

pwm = int(self.S.servo_target_pwm)

if self._last_servo_pwm_sent != pwm:

self._apply_servo_pwm_direct(pwm)

def do_release_overrides(self):

"""

Release overrides via QTimer pulses — no GUI blocking.

Sends NO_OVERRIDE on all 8 channels, 6 times at ~30ms intervals.

"""

if not self.m:

return

try:

if hasattr(self, "_release_timer") and self._release_timer.isActive():

self._release_timer.stop()

except Exception:

pass

self._release_count = 0

self._release_timer = QTimer(self)

self._release_timer.timeout.connect(self._release_tick)

self._release_timer.start(30)

def _release_tick(self):

"""One pulse of the non-blocking override release."""

if self._release_count >= 6 or not self.m:

self._release_timer.stop()

return

try:

self.m.mav.rc_channels_override_send(

self.m.target_system, self.m.target_component,

NO_OVERRIDE, NO_OVERRIDE, NO_OVERRIDE, NO_OVERRIDE,

NO_OVERRIDE, NO_OVERRIDE, NO_OVERRIDE, NO_OVERRIDE)

except Exception:

pass

self._release_count += 1

def do_abort(self, checked=False, reason=None):

"""

Stop the current GPS/DR go-to only.

Important PyQt note:

QPushButton.clicked emits a boolean "checked" argument. In v11.4 the

slot accepted a single positional parameter called "reason", so a GUI

click passed False into ap_status and QLabel.setText(False) crashed.

This handler now accepts the clicked(bool) argument safely while still

allowing internal calls like self.do_abort("GPS arrived ...") or

self.do_abort(reason="...").

v11.4.1 behaviour:

- no GUIDED mission is cancelled because none is created

- no forced TX hand-back is done

- if the override switch stays ON, Pi keyboard control remains available

immediately and ch1/ch2/ch4 simply return to neutral

"""

if isinstance(checked, str) and reason is None:

reason = checked

checked = False

if reason is None:

reason = "Go-to aborted — keyboard ready"

reason = str(reason)

with self.S.lock:

self.S.ap_active = False

self.S.ap_mode = APMode.NONE

self.S.ap_status = reason

self.S.locked = False

self.S.manual_backoff = False

self.S.force_tx_release = False

self.S.ap_roll_pwm = NEUTRAL

self.S.ap_pitch_pwm = NEUTRAL

self.S.ap_yaw_pwm = NEUTRAL

self.S.o_roll = NEUTRAL

self.S.o_pitch = NEUTRAL

self.S.o_yaw = NEUTRAL

self.S.k_roll = NEUTRAL

self.S.k_pitch = NEUTRAL

self.S.k_yaw = NEUTRAL

self.S.dr_phase = "idle"

self.S.dr_align_since = None

self.S.dr_last_update = None

self.S.auto_return_enabled = False

self.S.returning_to_start = False

self.S.return_context = ""

self.S.mission_start_lat = None

self.S.mission_start_lon = None

def _on_sw_ch_change(self, val):

global OVERRIDE_SWITCH_CH

OVERRIDE_SWITCH_CH = val

self.lbl_switch_pwm.setText(

f"ch{val} PWM: —  |  LOW<{SWITCH_LOW_PWM}  HIGH>{SWITCH_HIGH_PWM}")

def _clear_mission_memory(self):

with self.S.lock:

self.S.auto_return_enabled = True

self.S.returning_to_start = False

self.S.return_context = ""

self.S.mission_start_lat = None

self.S.mission_start_lon = None

self.S.payload_stage = "idle"

self.S.payload_stage_deadline = 0.0

def _begin_payload_sequence(self, mode_name):

with self.S.lock:

self.S.payload_stage = "open_hold"

self.S.payload_stage_deadline = time.time() + PAYLOAD_OPEN_SEC

self.S.servo_target_pwm = SERVO_OPEN_PWM

self.S.servo_status = "OPEN"

self.S.ap_roll_pwm = NEUTRAL

self.S.ap_pitch_pwm = NEUTRAL

self.S.ap_yaw_pwm = NEUTRAL

self.S.ap_status = f"{mode_name} arrived — payload OPEN for {PAYLOAD_OPEN_SEC:.0f}s"

self._last_servo_pwm_sent = None

def _start_return_leg(self, mode_name, cur_lat, cur_lon):

with self.S.lock:

already = self.S.returning_to_start

start_lat = self.S.mission_start_lat

start_lon = self.S.mission_start_lon

if already or start_lat is None or start_lon is None or cur_lat is None or cur_lon is None:

return False

if mode_name == "GPS":

back_dist = haversine_m(cur_lat, cur_lon, start_lat, start_lon)

if back_dist <= WAYPOINT_RADIUS_M:

return False

with self.S.lock:

self.S.ap_target_lat = start_lat

self.S.ap_target_lon = start_lon

self.S.returning_to_start = True

self.S.ap_roll_pwm = NEUTRAL

self.S.ap_pitch_pwm = NEUTRAL

self.S.ap_yaw_pwm = NEUTRAL

self.S.ap_status = f"GPS return -> {start_lat:.5f},{start_lon:.5f}  |  {back_dist:.1f} m"

return True

if mode_name == "DR":

back_dist = haversine_m(cur_lat, cur_lon, start_lat, start_lon)

if back_dist <= DR_PROGRESS_RADIUS_M:

return False

back_bearing = bearing_to(cur_lat, cur_lon, start_lat, start_lon)

with self.S.lock:

self.S.returning_to_start = True

self.S.dr_target_bearing = back_bearing

self.S.dr_target_dist_m = back_dist

self.S.dr_remaining_m = back_dist

self.S.dr_travelled_m = 0.0

self.S.dr_est_lat = cur_lat

self.S.dr_est_lon = cur_lon

self.S.dr_target_lat = start_lat

self.S.dr_target_lon = start_lon

self.S.dr_phase = "align"

self.S.dr_align_since = None

self.S.dr_last_update = time.time()

self.S.ap_roll_pwm = NEUTRAL

self.S.ap_pitch_pwm = NEUTRAL

self.S.ap_yaw_pwm = NEUTRAL

self.S.ap_status = f"DR return align -> {back_bearing:.0f}°  |  {back_dist:.1f} m"

return True

return False

# ══════════════════════════════════════════════════════════

#  MAP

# ══════════════════════════════════════════════════════════

def _on_map_click(self, lat, lon):

self._wp_lat = lat

self._wp_lon = lon

with self.S.lock:

self.S.ap_target_lat = lat

self.S.ap_target_lon = lon

self.inp_gps_lat.setText(f"{lat:.6f}")

self.inp_gps_lon.setText(f"{lon:.6f}")

self.map_view.page().runJavaScript(f"setWaypoint({lat},{lon});")

def _clear_wp(self):

self._wp_lat = None

self._wp_lon = None

self._B_bearing = None

self._B_dist = None

with self.S.lock:

self.S.ap_target_lat = None

self.S.ap_target_lon = None

self.inp_gps_lat.setText("— click map —")

self.inp_gps_lon.setText("—")

self.lbl_map_dist.setText("Dist: —")

self.map_view.page().runJavaScript("clearWaypoint();")

def _tick_map(self):

with self.S.lock:

lat = self.S.tel["lat"]

lon = self.S.tel["lon"]

hdg = self.S.tel["heading_deg"]

dr_lat = self.S.dr_est_lat

dr_lon = self.S.dr_est_lon

if hdg is None:

hdg = 0

show_lat = lat if (lat is not None and lat != 0.0 and lon is not None) else dr_lat

show_lon = lon if (lat is not None and lat != 0.0 and lon is not None) else dr_lon

if show_lat is not None and show_lon is not None:

self.map_view.page().runJavaScript(

f"setDrone({show_lat},{show_lon},{hdg:.1f});")

if self._wp_lat is not None and show_lat is not None and show_lon is not None:

d = haversine_m(show_lat, show_lon, self._wp_lat, self._wp_lon)

self.lbl_map_dist.setText(f"Dist: {d:.1f} m")

elif self._wp_lat is None:

self.lbl_map_dist.setText("Dist: —")

# ══════════════════════════════════════════════════════════

#  GPS AUTOPILOT START

# ══════════════════════════════════════════════════════════

def start_gps_autopilot(self):

with self.S.lock:

fix  = self.S.tel["gps_fix"]

tlat = self.S.ap_target_lat

tlon = self.S.ap_target_lon

sw   = self.S.override_active

cur_lat = self.S.tel["lat"]

cur_lon = self.S.tel["lon"]

if tlat is None or tlon is None:

QMessageBox.warning(self, "No Target",

"Click the map to set a waypoint first.")

return

if not fix or fix < 3:

QMessageBox.warning(self, "GPS Not Ready",

f"GPS fix type = {fix}  (need >= 3).")

return

if cur_lat is None or cur_lon is None or cur_lat == 0.0:

QMessageBox.warning(self, "GPS Not Ready",

"Current GPS position is not available yet.")

return

if not sw:

QMessageBox.warning(self, "Switch Not Active",

f"Flip TX ch{OVERRIDE_SWITCH_CH} HIGH to enable Pi control.")

return

if not self.m:

QMessageBox.critical(self, "No MAVLink", "Not connected to Pixhawk.")

return

auto_return = True

with self.S.lock:

self.S.force_tx_release = False

self.S.ap_mode   = APMode.GPS

self.S.ap_active = True

self.S.locked    = False

self.S.ap_roll_pwm  = NEUTRAL

self.S.ap_pitch_pwm = NEUTRAL

self.S.ap_yaw_pwm   = NEUTRAL

self.S.mission_start_lat = cur_lat

self.S.mission_start_lon = cur_lon

self.S.auto_return_enabled = auto_return

self.S.returning_to_start = False

self.S.return_context = "gps"

self.S.ap_status = (

f"GPS go-to -> {tlat:.5f},{tlon:.5f}  |  local RC-sim  |  auto-drop + auto-return")

def start_dr_autopilot(self):

with self.S.lock:

sw_on = self.S.override_active

hdg = self.S.tel["heading_deg"]

cur_lat = self.S.tel["lat"]

cur_lon = self.S.tel["lon"]

if not self.m:

QMessageBox.critical(self, "No MAVLink", "Not connected to Pixhawk.")

return

if not sw_on:

QMessageBox.warning(self, "Switch Not Active",

f"Flip TX ch{OVERRIDE_SWITCH_CH} HIGH to enable Pi control.")

return

if hdg is None:

QMessageBox.warning(self, "Heading Not Ready",

"Pixhawk heading is not available yet.")

return

mode = self._current_dr_mode()

bearing = None

dist = None

est_lat = None

est_lon = None

target_lat = None

target_lon = None

try:

if mode == DRInputMode.BEARING_DIST:

bearing = float(self.dr_A_bearing.text()) % 360.0

dist = float(self.dr_A_dist.text())

if cur_lat is not None and cur_lon is not None and cur_lat != 0.0:

est_lat, est_lon = cur_lat, cur_lon

elif mode == DRInputMode.COORDINATES:

if self._B_bearing is None or self._B_dist is None:

QMessageBox.warning(self, "Not Calculated", "Press Calculate first.")

return

start_lat = float(self.dr_B_slat.text())

start_lon = float(self.dr_B_slon.text())

target_lat = float(self.dr_B_elat.text())

target_lon = float(self.dr_B_elon.text())

bearing = float(self._B_bearing) % 360.0

dist = float(self._B_dist)

est_lat, est_lon = start_lat, start_lon

else:

QMessageBox.warning(self, "Mode Removed", "Dead Reckoning Mode C has been removed in v50.")

return

except ValueError:

QMessageBox.warning(self, "Input Error", "Enter valid dead reckoning values.")

return

if dist is None or dist <= 0.0:

QMessageBox.warning(self, "Input Error", "Distance must be greater than zero.")

return

auto_return = True

if target_lat is None or target_lon is None:

if est_lat is not None and est_lon is not None:

target_lat, target_lon = gps_offset(est_lat, est_lon, bearing, dist)

with self.S.lock:

self.S.force_tx_release = False

self.S.ap_mode = APMode.DR

self.S.ap_active = True

self.S.locked = False

self.S.ap_roll_pwm = NEUTRAL

self.S.ap_pitch_pwm = NEUTRAL

self.S.ap_yaw_pwm = NEUTRAL

self.S.dr_input_mode = mode

self.S.dr_target_bearing = bearing

self.S.dr_target_dist_m = dist

self.S.dr_remaining_m = dist

self.S.dr_travelled_m = 0.0

self.S.dr_start_lat = est_lat

self.S.dr_start_lon = est_lon

self.S.dr_est_lat = est_lat

self.S.dr_est_lon = est_lon

self.S.dr_target_lat = target_lat

self.S.dr_target_lon = target_lon

self.S.dr_phase = "align"

self.S.dr_align_since = None

self.S.dr_last_update = time.time()

self.S.mission_start_lat = est_lat

self.S.mission_start_lon = est_lon

self.S.auto_return_enabled = auto_return

self.S.returning_to_start = False

self.S.return_context = "dr"

self.S.ap_status = f"DR go-to align -> {bearing:.0f}°  |  {dist:.1f} m  |  auto-drop + auto-return"

# ══════════════════════════════════════════════════════════

#  LOCAL GO-TO TICK  (10 Hz)

#

#  BUG FIX (v11): heading=0 (north) is now handled correctly.

#  Previous code used `not all((..., hdg, ...))` which treated

#  heading 0 as falsy, silently killing the AP loop.

# ══════════════════════════════════════════════════════════

def _tick_ap(self):

with self.S.lock:

active = self.S.ap_active

ap_mode = self.S.ap_mode

sw_on = self.S.override_active

payload_stage = self.S.payload_stage

payload_deadline = self.S.payload_stage_deadline

return_ctx = self.S.return_context

returning = self.S.returning_to_start

if active and not sw_on:

self.do_abort("Aborted — switch flipped to TX")

return

if not active:

return

if payload_stage != "idle":

now = time.time()

with self.S.lock:

self.S.ap_roll_pwm = NEUTRAL

self.S.ap_pitch_pwm = NEUTRAL

self.S.ap_yaw_pwm = NEUTRAL

if payload_stage == "open_hold" and now >= payload_deadline:

with self.S.lock:

self.S.servo_target_pwm = SERVO_CLOSE_PWM

self.S.servo_status = "CLOSED"

self.S.payload_stage = "wait_return"

self.S.payload_stage_deadline = now + PAYLOAD_WAIT_SEC

self.S.ap_status = f"Payload closed — waiting {PAYLOAD_WAIT_SEC:.0f}s before return"

self._last_servo_pwm_sent = None

return

if payload_stage == "wait_return" and now >= payload_deadline:

with self.S.lock:

cur_lat = self.S.tel["lat"] if self.S.tel["lat"] not in (None, 0.0) else self.S.dr_est_lat

cur_lon = self.S.tel["lon"] if self.S.tel["lon"] not in (None, 0.0) else self.S.dr_est_lon

self.S.payload_stage = "idle"

self.S.payload_stage_deadline = 0.0

mode_name = "GPS" if return_ctx == "gps" else "DR"

if not self._start_return_leg(mode_name, cur_lat, cur_lon):

self.do_abort("Return start unavailable — ready for next command")

return

return

if ap_mode == APMode.GPS:

with self.S.lock:

lat = self.S.tel["lat"]

lon = self.S.tel["lon"]

hdg = self.S.tel["heading_deg"]

tlat = self.S.ap_target_lat

tlon = self.S.ap_target_lon

returning = self.S.returning_to_start

if lat is None or lon is None or hdg is None or tlat is None or tlon is None:

return

dist = haversine_m(lat, lon, tlat, tlon)

target_bearing = bearing_to(lat, lon, tlat, tlon)

yaw_err = wrap180(target_bearing - hdg)

if dist <= WAYPOINT_RADIUS_M:

if returning:

self.do_abort(f"GPS return arrived ({dist:.1f} m) — ready for next command")

else:

self._begin_payload_sequence("GPS")

return

yaw_pwm = clamp(NEUTRAL + yaw_err * AP_YAW_KP)

pitch_pwm = AP_PITCH_FWD if abs(yaw_err) < YAW_DEADBAND_DEG * 2 else NEUTRAL

with self.S.lock:

self.S.ap_roll_pwm = NEUTRAL

self.S.ap_pitch_pwm = pitch_pwm

self.S.ap_yaw_pwm = yaw_pwm

self.S.ap_status = (

f"GPS: {dist:.1f}m  brg={target_bearing:.0f}°  err={yaw_err:+.0f}°  "

f"{'FWD' if pitch_pwm != NEUTRAL else 'aligning'}")

return

if ap_mode != APMode.DR:

return

with self.S.lock:

hdg = self.S.tel["heading_deg"]

gs = self.S.tel["gs_mps"]

phase = self.S.dr_phase

bearing = self.S.dr_target_bearing

target_dist = self.S.dr_target_dist_m

travelled = self.S.dr_travelled_m

remaining = self.S.dr_remaining_m

est_lat = self.S.dr_est_lat

est_lon = self.S.dr_est_lon

target_lat = self.S.dr_target_lat

target_lon = self.S.dr_target_lon

align_since = self.S.dr_align_since

last_update = self.S.dr_last_update

returning = self.S.returning_to_start

if hdg is None:

with self.S.lock:

self.S.ap_roll_pwm = NEUTRAL

self.S.ap_pitch_pwm = NEUTRAL

self.S.ap_yaw_pwm = NEUTRAL

self.S.ap_status = "DR waiting for heading..."

return

yaw_err = wrap180(bearing - hdg)

if phase == "align":

yaw_pwm = clamp(NEUTRAL + yaw_err * AP_YAW_KP) if abs(yaw_err) > YAW_DEADBAND_DEG else NEUTRAL

pitch_pwm = NEUTRAL

now = time.time()

if abs(yaw_err) <= YAW_DEADBAND_DEG:

if align_since is None:

align_since = now

elif now - align_since >= YAW_ALIGN_SEC:

with self.S.lock:

self.S.dr_phase = "fly"

self.S.dr_align_since = None

self.S.dr_last_update = now

self.S.ap_status = f"DR fly -> {bearing:.0f}°  |  {target_dist:.1f} m"

return

else:

align_since = None

with self.S.lock:

self.S.dr_align_since = align_since

self.S.ap_roll_pwm = NEUTRAL

self.S.ap_pitch_pwm = pitch_pwm

self.S.ap_yaw_pwm = yaw_pwm

hold_txt = f"hold {max(0.0, YAW_ALIGN_SEC - (time.time() - align_since)):.1f}s" if align_since else "aligning"

self.S.ap_status = f"DR align: err={yaw_err:+.0f}°  |  {hold_txt}"

return

if phase != "fly":

with self.S.lock:

self.S.ap_roll_pwm = NEUTRAL

self.S.ap_pitch_pwm = NEUTRAL

self.S.ap_yaw_pwm = NEUTRAL

return

now = time.time()

dt = 0.0 if last_update is None else max(0.0, min(0.5, now - last_update))

speed = gs if (gs is not None and gs > DR_MIN_GS_VALID) else CRUISE_SPEED_MPS

step_m = speed * dt

travelled = min(target_dist, travelled + step_m)

remaining = max(0.0, target_dist - travelled)

if est_lat is not None and est_lon is not None and step_m > 0.0:

est_lat, est_lon = gps_offset(est_lat, est_lon, bearing, step_m)

arrived = remaining <= 0.0

if not arrived and target_lat is not None and target_lon is not None and est_lat is not None and est_lon is not None:

arrived = haversine_m(est_lat, est_lon, target_lat, target_lon) <= DR_PROGRESS_RADIUS_M

if arrived:

with self.S.lock:

self.S.dr_travelled_m = travelled

self.S.dr_remaining_m = 0.0

self.S.dr_est_lat = est_lat

self.S.dr_est_lon = est_lon

if returning:

self.do_abort(f"DR return arrived ({travelled:.1f} m) — ready for next command")

else:

self._begin_payload_sequence("DR")

return

yaw_pwm = clamp(NEUTRAL + yaw_err * AP_YAW_KP) if abs(yaw_err) > YAW_DEADBAND_DEG else NEUTRAL

pitch_pwm = AP_PITCH_FWD if abs(yaw_err) < YAW_DEADBAND_DEG * 2 else NEUTRAL

with self.S.lock:

self.S.dr_last_update = now

self.S.dr_travelled_m = travelled

self.S.dr_remaining_m = remaining

self.S.dr_est_lat = est_lat

self.S.dr_est_lon = est_lon

self.S.ap_roll_pwm = NEUTRAL

self.S.ap_pitch_pwm = pitch_pwm

self.S.ap_yaw_pwm = yaw_pwm

self.S.ap_status = (

f"DR: {travelled:.1f}/{target_dist:.1f} m  left={remaining:.1f} m  err={yaw_err:+.0f}°  {speed:.1f} m/s")

# ══════════════════════════════════════════════════════════

#  RC OVERRIDE SEND  (20 Hz, local stick simulation only)

#

#  ch3 (throttle) = NO_OVERRIDE always — TX owns throttle.

#  Servo is NOT sent through Pixhawk in v50; it is driven directly from Pi GPIO.

#  Switch OFF → all NO_OVERRIDE.

#  Switch ON + AP → computed AP stick values.

#  Switch ON + keyboard/idle → smoothed local stick values.

#  After operator abort, outputs return to NEUTRAL but Pi remains

#  immediately available for keyboard input while the switch stays ON.

#  Locked → NEUTRAL on ch1/ch2/ch4.

#  NEVER calls set_mode() or sends guided-navigation commands.

# ══════════════════════════════════════════════════════════

def _tick_send(self):

if not self.m:

return

dt = 1.0 / SEND_HZ

max_step = int(MAX_PWM_RATE * dt)

with self.S.lock:

sw_on = self.S.override_active

locked = self.S.locked

mb = self.S.manual_backoff

ap_active = self.S.ap_active

ap_mode = self.S.ap_mode

if not sw_on:

roll_pwm = pitch_pwm = yaw_pwm = NO_OVERRIDE

elif ap_active and ap_mode in (APMode.GPS, APMode.DR):

roll_pwm = self.S.ap_roll_pwm

pitch_pwm = self.S.ap_pitch_pwm

yaw_pwm = self.S.ap_yaw_pwm

elif locked:

roll_pwm = NEUTRAL

pitch_pwm = PITCH_BACKOFF_PWM if mb else PITCH_NEUTRAL_PWM

yaw_pwm = NEUTRAL

else:

def approach(o, t):

if o < t:

return min(o + max_step, t)

if o > t:

return max(o - max_step, t)

return o

self.S.o_roll = approach(self.S.o_roll, self.S.k_roll)

self.S.o_pitch = approach(self.S.o_pitch, self.S.k_pitch)

self.S.o_yaw = approach(self.S.o_yaw, self.S.k_yaw)

roll_pwm = self.S.o_roll

pitch_pwm = self.S.o_pitch

yaw_pwm = self.S.o_yaw

channels = [NO_OVERRIDE] * 8

channels[0] = int(roll_pwm)

channels[1] = int(pitch_pwm)

channels[2] = NO_OVERRIDE

channels[3] = int(yaw_pwm)

try:

self.m.mav.rc_channels_override_send(

self.m.target_system, self.m.target_component,

*channels)

except Exception:

pass

# ══════════════════════════════════════════════════════════

#  UI UPDATE TICK  (10 Hz)

# ══════════════════════════════════════════════════════════

def _tick_ui(self):

# ── Camera frame ──────────────────────────────────────

with self.S.lock:

frame = self.S.latest_frame

if frame is not None:

try:

fh, fw, fch = frame.shape

img = QImage(frame.tobytes(), fw, fh, fch * fw,

QImage.Format_RGB888)

pix = QPixmap.fromImage(img).scaled(

self.cam_label.width(), self.cam_label.height(),

Qt.KeepAspectRatio, Qt.SmoothTransformation)

self.cam_label.setPixmap(pix)

except Exception:

pass

# ── Read shared state ─────────────────────────────────

with self.S.lock:

tel         = dict(self.S.tel)

locked      = self.S.locked

detections  = list(self.S.latest_detections)

summary     = self.S.detection_summary

mb          = self.S.manual_backoff

k           = (self.S.k_roll, self.S.k_pitch, self.S.k_yaw)

o           = (self.S.o_roll, self.S.o_pitch, self.S.o_yaw)

ap_active   = self.S.ap_active

ap_mode     = self.S.ap_mode

ap_status   = self.S.ap_status

sw_on       = self.S.override_active

sw_pwm      = self.S.override_switch_pwm

rc_channels = dict(self.S.rc_channels)

force_release = self.S.force_tx_release

dr_phase    = self.S.dr_phase

dr_done     = self.S.dr_travelled_m

dr_total    = self.S.dr_target_dist_m

dr_left     = self.S.dr_remaining_m

dr_est_lat  = self.S.dr_est_lat

dr_est_lon  = self.S.dr_est_lon

dr_bearing  = self.S.dr_target_bearing

recording   = self.S.recording

record_path = self.S.record_path

record_start= self.S.record_started_at

record_error= self.S.record_error

servo_status= self.S.servo_status

payload_stage = self.S.payload_stage

returning   = self.S.returning_to_start

armed  = tel["armed"]

bv     = tel["bat_v"]

ba     = tel["bat_a"]

h      = tel["heading_deg"]

a      = tel["rel_alt_m"]

gs     = tel["gs_mps"]

fx     = tel["gps_fix"]

gps_ok = fx is not None and fx >= 3

# ── Horizon ───────────────────────────────────────────

self.horizon.update_attitude(tel["roll_deg"], tel["pitch_deg"])

# ── Override switch indicator ─────────────────────────

if sw_on and not force_release:

self.lbl_switch.setText("PI OVERRIDE ACTIVE  (ch1/ch2/ch4)")

self.lbl_switch.setStyleSheet(

f"font-size: 18px; font-weight: bold; color: {C_YELLOW};")

else:

self.lbl_switch.setText("TX FULL CONTROL")

self.lbl_switch.setStyleSheet(

f"font-size: 18px; font-weight: bold; color: {C_GREEN};")

self.lbl_switch_pwm.setText(

f"ch{OVERRIDE_SWITCH_CH} PWM: {sw_pwm}  |  "

f"LOW<{SWITCH_LOW_PWM}  HIGH>{SWITCH_HIGH_PWM}")

# ── Telemetry labels ──────────────────────────────────

self._tl["mode"].setText(tel["mode"])

self._tl["armed"].setText("ARMED" if armed else "DISARMED")

self._tl["armed"].setStyleSheet(

f"color: {C_RED if armed else C_DIM}; font-family: Consolas;")

self._tl["bat"].setText(

f"{bv:.2f} V  ·  {ba:.2f} A" if bv else "—")

if bv:

c = C_RED if bv < 10.5 else (C_YELLOW if bv < 11.0 else C_GREEN)

self._tl["bat"].setStyleSheet(f"color: {c}; font-family: Consolas;")

self._tl["nav"].setText(

f"{h:.0f}°  ·  {a:.1f} m  ·  {gs:.1f} m/s"

if h is not None else "—")

self._tl["gps"].setText(

f"fix={fx}  sats={tel['gps_sats']}  "

f"{tel['lat']:.6f}, {tel['lon']:.6f}"

if tel["lat"] else "—")

self._tl["gps"].setStyleSheet(

f"color: {C_GREEN if gps_ok else C_YELLOW};"

f" font-family: Consolas; font-size: 11px;")

if tel["ax"] is not None:

self._tl["imu"].setText(

f"x={tel['ax']:+.2f}  y={tel['ay']:+.2f}  z={tel['az']:+.2f}")

self._tl["thr"].setText(

f"{tel['thr_pwm']} PWM" if tel["thr_pwm"] else "—")

# RC channels

sw_ch = OVERRIDE_SWITCH_CH

if rc_channels:

parts = []

for ch in range(1, 10):

v = rc_channels.get(ch, 0)

if v > 0:

tag = " ◄" if ch == sw_ch else ""

parts.append(f"ch{ch}:{v}{tag}")

self._tl["rc"].setText("  ".join(parts) if parts else "waiting…")

else:

self._tl["rc"].setText("waiting…")

# ── Control state ─────────────────────────────────────

if force_release or not sw_on:

self.lbl_state.setText("TX IN CONTROL")

self.lbl_state.setStyleSheet(

f"font-size: 16px; font-weight: bold; color: {C_GREEN};")

elif ap_active and ap_mode == APMode.GPS:

self.lbl_state.setText("GPS AUTOPILOT")

self.lbl_state.setStyleSheet(

f"font-size: 16px; font-weight: bold; color: {C_CYAN};")

elif ap_active and ap_mode == APMode.DR:

self.lbl_state.setText("DEAD RECKONING")

self.lbl_state.setStyleSheet(

f"font-size: 16px; font-weight: bold; color: {C_CYAN};")

elif payload_stage != 'idle':

self.lbl_state.setText("PAYLOAD ACTION")

self.lbl_state.setStyleSheet(

f"font-size: 16px; font-weight: bold; color: {C_ORANGE};")

else:

self.lbl_state.setText("KEYBOARD ACTIVE")

self.lbl_state.setStyleSheet(

f"font-size: 16px; font-weight: bold; color: {C_ACCENT};")

self.lbl_scene.setText(f"Detected: {summary}")

if recording:

secs = max(0, int(time.time() - record_start)) if record_start else 0

self.lbl_record.setText(f"Recording: ON  ·  {secs}s")

self.btn_record.setText("STOP RECORDING")

else:

self.lbl_record.setText(f"Recording: OFF{'  ·  ' + record_error if record_error else ''}")

self.btn_record.setText("START RECORDING")

backend_text = self.servo_hw.describe()

if getattr(self.servo_hw, "error", "") and self.servo_hw.backend == "unavailable":

backend_text += f"  ·  {self.servo_hw.error}"

self.lbl_servo.setText(f"Servo: {servo_status}")

self.lbl_servo_hw.setText(backend_text)

self.lbl_targets.setText(f"Target   R={k[0]}  P={k[1]}  Y={k[2]}")

self.lbl_outputs.setText(f"Output   R={o[0]}  P={o[1]}  Y={o[2]}")

self.lbl_ap_status.setText(str(ap_status))

# Dead reckoning panel

if hasattr(self, "lbl_dr_status"):

self.lbl_dr_status.setText(str(ap_status) if ap_mode == APMode.DR or dr_total > 0 else "Status: Idle")

progress_pct = 0

if dr_total > 0:

progress_pct = int(max(0.0, min(100.0, (dr_done / dr_total) * 100.0)))

self.dr_pbar.setValue(progress_pct)

self.lbl_dr_prog.setText(f"{dr_done:.1f} / {dr_total:.1f} m  |  left={dr_left:.1f} m" if dr_total > 0 else "0.0 / 0.0 m")

self.lbl_dr_speed.setText(f"Speed: {gs:.1f} m/s" if gs is not None else f"Speed: {CRUISE_SPEED_MPS:.1f} m/s (est)")

if dr_est_lat is not None and dr_est_lon is not None:

self.lbl_dr_pos.setText(f"Estimated position: {dr_est_lat:.6f}, {dr_est_lon:.6f}")

else:

self.lbl_dr_pos.setText("Estimated position: —")

if hasattr(self, "lbl_C_live"):

if h is not None:

err = wrap180(dr_bearing - h) if ap_mode == APMode.DR or dr_total > 0 else 0.0

self.lbl_C_live.setText(f"Current heading: {h:.0f}°   |   Align error: {err:+.0f}°   |   phase={dr_phase}")

else:

self.lbl_C_live.setText("Current heading: —   |   Align error: —")

# ── Status strip ──────────────────────────────────────

self._mb["mode"].setText(f"MODE: {tel['mode']}")

self._mb["armed"].setText("ARMED" if armed else "DISARMED")

self._mb["armed"].setStyleSheet(

f"color: {C_RED if armed else C_DIM};"

f" font-family: Consolas; font-size: 11px; font-weight: bold;")

sw_c = C_YELLOW if sw_on else C_GREEN

self._mb["sw"].setText(f"SW: {sw_pwm} ({'ON' if sw_on else 'OFF'})")

self._mb["sw"].setStyleSheet(

f"color: {sw_c}; font-family: Consolas; font-size: 11px; font-weight: bold;")

self._mb["bat"].setText(f"BAT: {bv:.1f}V" if bv else "BAT: —")

if bv:

bc = C_RED if bv < 10.5 else (C_YELLOW if bv < 11.0 else C_GREEN)

self._mb["bat"].setStyleSheet(

f"color: {bc}; font-family: Consolas; font-size: 11px;")

self._mb["hdg"].setText(f"HDG: {h:.0f}°" if h is not None else "HDG: —")

self._mb["alt"].setText(f"ALT: {a:.1f}m" if a else "ALT: —")

self._mb["gs"].setText(f"GS: {gs:.1f}" if gs else "GS: —")

self._mb["gps"].setText(

f"GPS: {tel['gps_sats']}sat" if tel["gps_sats"] else "GPS: —")

self._mb["gps"].setStyleSheet(

f"color: {C_GREEN if gps_ok else C_YELLOW};"

f" font-family: Consolas; font-size: 11px;")

ap_c = C_CYAN if ap_active else (C_GREEN if force_release else C_DIM)

self._mb["ap"].setText(f"AP: {str(ap_status)[:40]}")

self._mb["ap"].setStyleSheet(

f"color: {ap_c}; font-family: Consolas; font-size: 11px;"

f" font-weight: {'bold' if ap_active else 'normal'};")

# ══════════════════════════════════════════════════════════

#  CLOSE

# ══════════════════════════════════════════════════════════

def closeEvent(self, event):

with self.S.lock:

self.S.stop      = True

self.S.ap_active = False

self.S.force_tx_release = True

self.S.recording = False

self.S.servo_target_pwm = SERVO_CLOSE_PWM

self.S.servo_status = "CLOSED"

if self._mav_worker:

self._mav_worker.stop()

if self.m:

try:

for _ in range(4):

self.m.mav.rc_channels_override_send(

self.m.target_system, self.m.target_component,

NO_OVERRIDE, NO_OVERRIDE, NO_OVERRIDE, NO_OVERRIDE,

NO_OVERRIDE, NO_OVERRIDE, NO_OVERRIDE, NO_OVERRIDE)

time.sleep(0.03)

except Exception:

pass

try:

self._apply_servo_pwm_direct(SERVO_CLOSE_PWM)

self.servo_hw.close()

except Exception:

pass

if self._mav_thread:

self._mav_thread.quit()

self._mav_thread.wait(2000)

event.accept()

# ─────────────────────────────────────────────────────────────

#  MAIN

# ─────────────────────────────────────────────────────────────

def main():

S = SharedState()

app = QApplication(sys.argv)

app.setStyle("Fusion")

# Start camera in background

threading.Thread(target=camera_thread_fn, args=(S,), daemon=True).start()

# Light palette

pal = QPalette()

pal.setColor(QPalette.Window,          QColor(C_BG))

pal.setColor(QPalette.WindowText,      QColor(C_TEXT))

pal.setColor(QPalette.Base,            QColor(C_SURFACE))

pal.setColor(QPalette.AlternateBase,   QColor(C_BG))

pal.setColor(QPalette.Text,            QColor(C_TEXT))

pal.setColor(QPalette.Button,          QColor(C_SURFACE))

pal.setColor(QPalette.ButtonText,      QColor(C_TEXT))

pal.setColor(QPalette.Highlight,       QColor(C_ACCENT))

pal.setColor(QPalette.HighlightedText, QColor(C_BG))

app.setPalette(pal)

win = GCSWindow(S)

win.show()

sys.exit(app.exec_())

if __name__ == "__main__":

main()